AA4RE BBS and the G8BPQ Switch.

The AA4RE BBS can now be used via the DEDHOST Driver. See file DEDHOST.DOC

for full details of DEDHOST. The BBS must be configured as if you are using

a DRSI card, but you shouldnt load TNCTSR. You must run BBSETTNC before BB.

My testing was with AA4RE version 2.10, using parameters given below.

In PARAM.BB:

port

port_type       = PC*PA-0

com             = 255

max_pac         = 128

speed           = 4800

max_connect     = 8

max_channel     = 9

max_pending     = 3

monitor         = YES

time_out        = 180

fwd_min         = 15

bbs_only        = N

remote_sysop    = Y

upload_download = Y

broadcast       = y			; sends mail beacon on all radio ports

port_name       = 144.650/432.675 NODE

broadcast_path  = MAIL		; is ignored - set in config.txt file

monitor_color   = 121

no_out_fwd      = N

endport

In ROUTE.BB

G8BPQ  LEAVE

@BLANK LEAVE

*      GB7BAD

In PATH.BB

PATH GB7BAD A GB7BAD FORCE

T 0000 2300

C SWITCH				; note initial C SWITCH

SC GB7BAD

R* to *

Startup File.

The 4th parameter to DEDHOST is the buffer space allocated to DEDHOST. The

value you need will depend on traffic, and you may need to experiment to

find the optimum value for your installation. The mimimum is 8, and the

maximum 60. The more space you allocate here, the less will be available to

the BBS software.

BPQCODE

DEDHOST 9 1 1 32 255

BBSETTNC

BB

If you are using DesqView, BPQCODE should be loaded before DV, and DEDHOST

should be loaded in the AA4RE window.

John Wiseman, G8BPQ.

16/3/91

revised 9/2/92

###############