TSTHOST v1.42 (c) IK1GKJ                                     1

TstHost version 1.42

--------------------

Features

--------

TSTHOST   is  a software  written for  TNC2 and  clones with   HOST

mode  EPROM type  TF8, TF23,  TF24 etc.  It is possible  to use the

software with other TNCs set in KISS mode.

A  TNC in  KISS mode  requires a  serial driver (like TFPCX/TFPCR),

but  don't forget  to set  the TNC  in  KISS  mode  BEFORE  running

TSTHost.

A  TNC in  WA8DED Host  mode  requires  a  serial    driver    like

COMBIOS,  MBBIOS etc.  The  GKJBIOS    driver    is    an    MBBIOS

compatible   driver  optimised for   TSTHost,  that   detects   the

special   16550A  UART.   For  the    driver    configuration,  run

GKBIOCFG.EXE and follow the instructions displayed on the screen.

TSTHOST  offers from  2 to  8 independent  communication  channels,

where   every  channel has  a 300  lines receive  buffer,  circular

keyboard   buffer  to recall  the ten  previous commands, automatic

text  justification, up   to     8 simultaneous  YAPP (with  resume

and   yappC  options) or  ASCII file  transfers,   up  to    8  PMS

channels   with RLI-type forward with  a master  BBS. All  received

data  can   be   saved   in  a  disk file. Every session may have a

different callsign.

There   is   a  separated window for monitoring, and it is possible

to capture in a file all the traffic displayed in that window.

The  text entered  from the  keyboard is  recognised as commands to

run  for every  COMMAND mode session, while in CONVERSATION mode is

sent  to   the   connected   station.   For this  reason it  is not

possible  to switch  in conversation  mode if  the session  is  not

connected.  The exception     is   the  Monitor  window,  where all

keyboard data  is transmitted as Unproto packets.

For   every  session   there is  a receive buffer, useful to review

all  the received  data. By  receiving data  from   a  channel  not

actually  selected, the  related callsign  in the  status line will

blink.   All  unconnected  channels  are  set  by  default  to  PMS

sessions.  So, when  an user  connects your  station, he  goes into

the   personal  mailbox system,  but if  YOU  connect  a  user  the

channel  is set   for  a normal  conversation mode.  Naturally, you

have  a specific command to switch  a PMS session into conversation

mode  and vice-versa.  During a  PMS session, the user can read and

write  messages, view  the users directory, up or download programs

TSTHOST v1.42 (c) IK1GKJ                                     2

with  the YAPP  protocol. The   PMS  session can  also run external

server   programs,   send   a   Break  to other PMS users, talk the

sysop.

Incoming  connection in  a PMS  windows, may  be  managed  from  an

external filtering program, called CON_FILT.EXE

TSTHost   automatically  recognises the  standard password  format,

and  the   extended MD2   algorithm.  The  standard password format

accepts  from   one to   ten  letters,    but  normally    all  the

programs require five letters for the password.

Other features are the Unproto list capability and a full forward

protocol  MBL-RLI or  FBB COMPRESSED  type, even  if  the  the  BBS

require a password for the access.

The  program can  also extract  any 7plus  code from the unproto or

pms   messages,  and   write  this  code  to  a  file,  for  furter

processing.  TstHost support remote SERVER technique and PG program

to increase the pms command for particoular applications.

At  the end,  is supported  also the  Macro Commands,  that may  be

active for any channels, without restriction.

The  communication in   HOST-mode   between  the PC and the TNC has

a  3 seconds  timeout. After  this time,  if the  program does  not

receive  a reply  from the  TNC, attempts to synchronise the serial

port.

TSTHOST v1.42 (c) IK1GKJ                                     3

Tsthost with TNC in KISS or BAYCOM modem  (/K)

----------------------------------------------

You   need  the driver  TFPCR or  TFKISS for  KISS mode,  TFPCX for

modem  BAYCOM.   TFPCX version 2.10 supports both the KISS and  the

BAYCOM modes.

Using  a KISS-mode   TNC,  set the  TNC in  KISS mode  manually  or

with   the     appropriate  command     line    switch    of    the

TFPCR/TFPCX/TFKISS driver.

TFPCX  uses the  IRQ 253  for the  internal  communications,    the

other drivers uses the IRQ 254.

Load the driver

Load TSTHost with the command:  TSTHOST /T /I254 (or /I253)

/T  sets TSTHost   to   use   the  TFPCX/R  driver, and  /I254 sets

the IRQ for the driver communication.

NOTE:  when started,  TSTHost automatically  sends the  commands to

the driver:  JHOST1, USERS NbrOfChannels, @U0

Example for TFPCR or TFKISS:

TFKISS command line parameters

TSTHOST /T /I254

Example for TFPCX:

TFPCX command line parameters

TSTHOST /T /I253

WARNING:   I  have noticed   that  TFPCR   version 3.30   does  not

work  properly with  high speed transfers over 400  cps (G3RUH 9600

baud).  I don't  know the  reason, but  version 2.10  of TFKISS and

TFPCX  does not have this bug.

TSTHOST v1.42 (c) IK1GKJ                                     4

Tsthost with the standard HOST mode  (/H)

-----------------------------------------

Configure  GKJBIOS or  other  COMBIOS  drivers  for  the    correct

serial port parameters. Load the driver in memory

Load TSTHost with the command TSTHOST /H /Cx /Bzzzz

where   /H  selects the  software to use the  standard host mode, x

is  the   serial port   where  you   have connected the TNC (1..8),

zzzz  is the  baud rate  of  the  port.  TSTHost  supports  a  data

transfer rate of 9600, 19200 and 38400 baud.

NOTE:  when started,  TSTHost automatically  sends the  commands to

the TNC:  JHOST1, USERS NbrOfChannels, Z0, @U0

At  the end   of  the   program, you  must remove  from memory  the

serial driver with the command:  GKJBIOS U

For instance:

GKJBIOS

TSTHOST /H /C1 /B19200

GKJBIOS U

NOTE:  You may  replace the  gkjbios or  any type of combios driver

with  the FOSSIL  type driver,  the program recognize automatically

the driver and use it.

Tsthost with DRSI driver  (/D)

------------------------------

This  interface permit  to use  tsthost with RSI driver that return

the  data in the wa8ded host compatible mode, laki DEDHOST provided

with BPQ.

- Configure your drsi driver for your work condition

- Load TstHost with the command TSTHOST /D /Ixx, where:

/D    tell tsthost to use a drsi driver

/Ixx  XX is the communication vector used by the driver, and must

be given in DECIMAL mode.

NOTE:  when started,  TSTHost automatically  sends the  commands to

the driver:  JHOST1, USERS NbrOfChannels, @U0

TSTHOST v1.42 (c) IK1GKJ                                     5

Tsthost with 2 or more tnc

--------------------------

You   may  connect   two or   more  interfaces to the program. This

can  be done with the TFPCX v2.10 driver. This  driver supports  up

to  8 interfaces   like  KISS,  BAYCOM,  MODEM  etc. This is only a

small  description,  because  the  complete  documentation  of  the

commands  is shipped with the driver files.

Since   the  driver   may support  up to  8 port,  the driver  have

a  command to  define which  channels assign  to  which  port.  The

extended  host command is @PO xxxxxxxx. Each 'x' is a value between

0  and the  number   of   installed   interfaces.   0 is  the first

interface,  1 the  second, etc. TSTHost uses 8 channels so you must

define 8 digits.

Of  course,   if you   use  the   driver for   one  interface only,

the  @PO  command  is  not  requested,  because  all  channels  are

assigned  to that interface.

Here are some examples:

1)   We  have two  TNCs,   connected to  COM1 and  COM2.  The  TNCs

are  already   set in  KISS mode,  the baud  rate is 19200 baud, we

want  to assign   channels  1234  to the TNC on COM1, channels 5678

to the TNC on COM2.

TFPCX -PKISS1 -PKISS2 -B19200:19200 -CH8

TSTHOST /T /I253

To assign the channels type the command:   PARAM 0 @PO 00001111

(This command can also be written in the tsthost.cfg file)

2)   We  have 2  TNC and  a  BAYCOM  modem.  The  BAYCOM  modem  is

connected  to COM1, 1200 baud; a TNC is on COM2 at 19200 bauds, the

second  TNC is   on  COM3,   IRQ10, 38400  baud. The  two TNCs  are

already  set in  KISS mode.  We to  want assign channels 123 to the

BAYCOM, 456 to the first TNC, and 78 to the second TNC.

TFPCX -PCOM1 -PKISS2 -PKISS3:03E8:10 -B1200:19200:9600 -CH8

TSTHOST /T /I253

To assign the channels, type the command PARAM 0 @PO 00011122

(This command can be executed from the tsthost.cfg file)

3)   A  TNC in  KISS mode,  already set  in   KISS mode,  on  COM1,

19200 baud.

TFPCX -PKISS1 -B19200 -CH8

TSTHOST /T /I253

In this case the command @PO is not requested.

TSTHOST v1.42 (c) IK1GKJ                                     6

Memory usage

------------

TstHost  use the  memory dinamically.  This means  that the  memory

will  be request  to the  system when needed, and released when not

more  requested. But,  some type  of data, like revision buffer and

channel structure, cannot be released.

Actually  tsthost need about 500 Kbyte of memory to run, this for a

standard  use, and  all pc that have installed almost 640 kbyte may

run tsthost. But others memory usage may be needed:

1) TFPCX-TFPCR, if used, need about 64K

2) Unproto list, (little mode), require 24K

3)  RUN command,  and generally  any  other  application  like  the

SERVER  that require  a shell  to operating system, need memory for

command.com and the program to execute.

4)  Compressed data  received with  forward and uproto list require

other 19K.

Expanded memory and /NOEMS command

----------------------------------

TstHost  detect and  use EMS  memory if  found in  the sistem.  EMS

memory may free up to 240Kbyte of conventional memory.

If  you   have any  problem using  the  expanded  memory,  you  can

disable  the function  with the  command line  switch  /NOEMS.  For

example:                      TSTHOST /H /B19200 /C1 /NOEMS

Extended memory and /NOXMS command

----------------------------------

TstHost  detect and  use XMS  memory if  found in  the sistem.  XMS

memory  may free  up to  210Kbyte of  conventional memory,  and  is

accessed  via the  HYMEM.SYS driver. (or equivalent). Note that XMS

memory management is slow referred to EMS and conventional memory.

TstHost  priority are:  first fill  in EMS,  next if needed in XMS,

next if needed in conventional memory.

Command  line parameters  /NOXMS  tell  thsthost  to  not  use  XMS

memory, for example:          TSTHOST /H /B19200 /C1 /NOXMS

TSTHOST v1.42 (c) IK1GKJ                                     7

Reduction of RX buffers, /Rsize command

---------------------------------------

If  you do  not have memory for all your applications, you may free

memory  reducing the  size of  the rx  buffer. Normally  any buffer

have  a capacity  of 300  lines. The  /Rsize permit  to define  the

number  of line  allocated in  the review  buffer, from 100 to 300.

Any line cost 82 bytes.

For example, with TSTHOST .... /R200, you may free:

(300-200)x82x9=73800 byte, 73K

This  command do  not affect  buffer allocated  in EMS memory, that

have always 300 line.

Number of channels, /K parameters

---------------------------------

This  parameter is  useful to reduce the numeber of channel used in

tsthost.  Application may be in old eprom that not manage more of 4

channel,  or to  reduce the  memory usage.  Is possible  to  define

from2  to 8  channel, 8  default: for example, to set up 4 channel,

add the parameters /K4 to the tsthost command line.

Unproto list size, /U parameters

--------------------------------

Tsthost  always attempt  to  set  the  unproto  list  size  to  800

messages.  If you  have memory occupation problems, this parameters

is  useful to  reduce the  memory usage: Available value start from

200  to 800,  800 default:  To set the unproto list size to at most

400  messages, add  the parameters  /U400 to  the  tsthost  command

line.

VGA mode

--------

Add  the parameter  /VGA to  the command  line, to  get  43  or  50

screen lines on an EGA or VGA monitor.

Example: TSTHOST /H /B19200 /C3 /VGA

TSTHOST v1.42 (c) IK1GKJ                                     8

Extended interface for data request, switch /V

----------------------------------------------

Starting  from version  1.41 TstHost  will provide  a new method to

access  particoular information about the status of the program and

the   tasks.  Extended  data  request  will  be  done  trought  the

internally  irq service  vector, normally  101, 65Hex.  This vector

may  be redefined  with the  command line parameter /V, that accept

in input a DECIMAL value. For example:

TSTHOST ... /V100

Tsthost tell tsthost to use irq number 100, 64Hex, instead of 101

NOTE: TSTHOST REQUIRE AN EXCLUSIVE ACCESS TO THIS VECTOR.

Read also the section about TSTHOST.IRQ vector.

Programs,  servers, pgs,  may interrogate  this  irq  in  order  to

obtain  info about  general parameters of tsthost and or tasks. The

info  will be  returned in  a structure  here described,  that is a

static  area into  tsthost. For this reason, if you need to mantain

information  between two  or more  calls to  the irq, you must copy

the  previsuly information  in an area of your program, because the

new irq call will cause the overwrite of the previusly info.

Program  that require info, must be call the irq vector with the AL

register set to 0, AH set to the specified channel to investigate.

When  return from  the  irq  routine,  if  any  error  was  detect,

register  AH is  set to not 0 value, and AL will contain the number

of  channel supported  by tsthost.  Normally AL is 8, but it may be

loss  if tsthost  was start  with the  /K switch in order to reduce

the  number of  channels. Actually  the unique error possible is to

investigate a channel that not exist.

If  success, register  AH will  be 0,  AL will  contain  again  the

maximum  number of  available channels, and ES:BX will point to the

info structure.

NEVAR,  NEVAR AND  NEVAR WRITE  INTO THIS  AREA. YOU MAY ONLY READ.

WRITING  INTO THIS  AREA PROBABLY  WILL  CAUSE  THE  CRASH  OF  THE

SYSTEM!

TSTHOST v1.42 (c) IK1GKJ                                     9

Here   is  the   format  of   the  information  structure.  Like  C

convention, all char array are null terminated.

struct info

{         // THIS VALUE ARE GLOBALS, NOT CHANNEL DEPANDANT.

//===============================================

unsigned char THVH;    //TstHost version, high value

unsigned char THVL;    //TstHost version, low value

int  MAXCHANNEL;       //Number of channel available i tsthost

char DRVTYPE;          //Driver type, 1 real host, 0 tfpc, 2 drsi

char PORT;             //If real host, com port

unsigned int BAUDRATE;//If real host, baudrate

int  INTNO;            //If tfpcx/r, irq vector used by driver

char TstHostCall[10];  //Callsign of the sistem, with ssid

char UListEnable;      //If not 0, unproto list is active

char Wpath[81];        //tsthost WorkDir

char Upath[101];       //tsthost UserDir. If more than one path

//is defined, the multiple path are

//separated by a space.

char HomeBbs[10];      //HomeBbs Callsign

char HomeAlias[10];    //homebbs alias call, null if undefined

// THIS FIELDS ARE CHANNEL DEPANDANT

//====================================================

int  chstatus;         //0 channel is disconnected

//1 standard connection, I have connect

//  another OM

//2 PMS connection, a remote user is

//  connected on my pms

//3 PMS connection, HomeBbs have connect

//  my pms to do forward.

//4 PMS connection, my pms have connect

//  HomeBbs to do forward

//5 UNPROTO connection, i have connect

//  HomeBbs to request unproto mail.

char SuppCall[10];     //If not null, extra callsign for the

//channel (command AX PORT)

char UserCall[10];     //call of the connected station, with ssid

// THIS FIELDS ARE VALID ONLY FOR USER THAT HAVE CONNECT

// MY PMS, chstatus=2 o 3

//=====================================================

char UIname[13];       //User name

long UILastConnTime;   //in sec dal 1970, last connection date

long UILastMsgList;    //in sec, last messaged listed date

long UINbrConn;        //number of connection for this user

long UIThisConnTime;   //in second, this date at connection

unsigned int SysFlag;  //actual SYS flag for the user

};

TSTHOST v1.42 (c) IK1GKJ                                     10

Short cut keys

--------------

ALT F1             Automatic transmission of the password.

ALT F2             Displays the MHEARD list

(depending on the driver/TNC software)

ALT F3             Enables the monitor traffic to IUSC

ALT F4             Disables the monitor traffic

ALT F5             Set monitor traffic to U

ALT F6             Like the command ULIST VIEW

ALT F7             Enable the file selection for file view

ALT F8             Display the message in the pms list

ALT F10            Turn off the new mail indicator.

CONTROL ESC or F10 Switches command<>conversation mode.

PAGE UP            Activates review buffer, or if already active

displays the previous page.

PAGE DOWN          Displays next page in review buffer

SHIFT UP           One line up in review mode

SHIFT DOWN         One line down in review mode

CONTROL PAGE DOWN  Quit from review mode

CONTROL HOME       Top of review buffer

CONTROL END        Bottom of review buffer

ENTER              In command mode: executes the command.

In conversation mode: transmits the text.

In both mode, if the revision buffer is active,

it will be disabled.

CURSOR UP          Recalls the previous line from kbd buffer.

CURSOR DOWN        Recalls the next line from the kbd buffer.

F1...F8            Switches to session 1 to 8

F9                 Switches to the Monitor window

CTRL-F1..F10       User defined.

SHIFT-F1..F10      User defined.

CTRL_BACKSPACE     Clears from cursor to the previous word.

CTRL_LEFT          Moves the cursor one word to the left

CTRL_RIGHT         Moves the cursor one word to the right

HOME               Moves the cursor at the beginning of the line

END                Moves the cursor at the end of the line

BACKSPACE          Deletes char to the left of the cursor

DELETE             Deletes char under the cursor

INSERT             Inserts mode toggle

ESCAPE             Clears the current line

CTRL-Y             Deletes the current line (internal ).

TSTHOST v1.42 (c) IK1GKJ                                     11

Path of the files

-----------------

There  isn't a  particular directory  for the TstHost installation,

but  you MUST  create, within  that directory, three subdirectories

with the names: MAIL, PG, USERS.

This can be a standard installation:

\TSTHOST         <DIR>

|-PG           <DIR>

|-MAIL         <DIR>

|-USERS        <DIR>

|-UPLOAD    <DIR>

|-TSTHOST.EXE

|-TSTHOST.DOC

|-TSTHOST.PSW  File for automatic password

|-TSTHOST.CFG  Configuration file for the program

|-TSTHOST.ENT  Input text when connect the pms

|-TSTHOST.HLP  Help file for pms

|-TSTHOST.SYS  Password file for remote pms access.

|-TSTHOST.PGS  List/description of the available PG server

|-TSTHOST.USR  User database, managed by tsthost

|-TSTHOST.IRQ  Irq identification, managed by tsthost

|-???????.ULS  Unproto list file, created/used by TSTHOST

|-???????.CON  Connection file for Homebbs, created by the user

|-TSTHOST.HBB  Automatic routing file

|-TSTHOST.SHD  Tnc configuration file when shutdown program

|-TSTHOST.CRN  Command file executed every hours.

|-TSTHOST.IN   Mail import file

|-TSTHOST.LOG  created from TSTHOST if LOG=ON

|-TSTHOST.BID  Bid file, managed by TSTHOST

|-TSTHOST.EXT  File estension definition, created by the user

|-TSTHOST.MAC  Macro definition file, created by the user

|-TSTHOST.DMS  PMS messages database, created/used by TSTHOST

|-DBFMSG.OLD   Old messages database, created/used by TSTHOST

|-Files for source command (if you create source files)

Detailed description of files and directory:

PG - Server directory

You  must place the PMS server in this directory. If this directory

does not exist, the PMS PG command does not work.

The next section of the manual describes the PG Server.

TSTHOST v1.42 (c) IK1GKJ                                     12

MAIL - Messagges directory

The  bulletins/personal   messages of  the PMS  will be  stored  in

this  directory. If  this path  does not exist, the electronic mail

section of the PMS does not work properly.

USERS - Users directory

This  is the  directory   that   the  users  access with  the W, YU

YD  commands. This  is the   only   path   that  can point to other

parts  of your   hard  disk.   If not  otherwise specified, TSTHost

uses  the path  USERS from  the directory   where   tsthost.exe  is

loaded.   You   can  specify any  other  drive/directory  with  the

USERDIR command.

UPLOAD - Users upload directory

This  is the  directory where will be stored any file uploaded by a

pms  users  with  the  YU  command.  If  you  do  not  create  this

directory, the YU PMS command will not work.

WORKDIR - Standard work directory

The  default value  is the  directory where  tsthost.exe is loaded,

but  you can  select any  other drive/directory  with  the  WORKDIR

command.  This path is used for the WRITEBUF, RECORD,  SEND,  YPUT,

YREC  and  MAILCOPY  commands.  With  these  commands,  unless  the

filename  does not contain a  specified  drive  and/or  path,   the

file   will   be   accessed in  the  WORKDIR  path.  For  instance,

by   executing   the  command WRITEBUF  TEXT, the  contents of  the

review  buffer will  be written  in   a file   called  TEXT  in the

directory  that  you have specified with  the WORKDIR  command, but

WRITEBUF  E:\ASC\TEXT will  write the buffer  into  a  file  called

TEXT   in  E:\ASC. The  purpose of  this command is to separate the

tsthost files  in  capture, YAPP receive and the other files.

TSTHOST.IRQ - Internal interrupt detection

Due  to the  new interface for request data in tsthost, that use an

irq  vector  to  request  the  info,  program  that  use  this  new

interface must known the vector used by tsthost.

This  file is  created when tsthost start, and deleted when tsthost

ends,  and its  unique purpose  is to tell the program that use the

new  tsthost interface  about  the  irq  vector  used  by  tsthost.

Internally,  this file,  that reside  in the tsthost.exe directiry,

have only this line:

TstHostInterruptVector=XX

TSTHOST v1.42 (c) IK1GKJ                                     13

Where  XX is a decimal number that is the irq used by tsthost. This

file  is important,  so nevar  put this  file READ  ONLY ann  nevar

manipulate the contents written by tsthost.

TSTHOST.USR - Users database

This  sistem file  is managed  from tsthost, and are created in the

tsthost.exe  directory. Actually,  the program  store in  this file

only the name of the user and the date of the last connection.

This is the format:

struct UInfo

{

char call[7];       /* user callsign */

char name[13];      /* user name */

long LastConnTime;  /* in sec. from 1970, last conn date */

long LastMsgList;   /* date of the last message listed */

long NbrConn;       /* number of connection fo this user */

long ThisConnTime;  /* data at connection time */

char PrivDir[31];   /* user private directory */

char reserved[61];  /* riserved */

};                  /* 128 byte every record */

TSTHOST.HBB - Automatic routing file

This  simple ascii  file, managed  directly from  sysop,  have  the

purpose  to assign  an automatic  routing for all locally generated

message  that do  not have  one.  Messege  that  have  a  specified

routing will not be althered.

The  format of  the file  is simple: Any invalid line is ignored. A

valid  line have  the format  CALL @  BBS, where  CALL and  BBS are

callsign WITHOUT ssid.

If  there is, for example, a line like this: IK1MSL @ IK1ZZZ, and i

write  a message  with SP  IK1MSL @ IK1ABC, the message will not be

tuched, but, SP IK1MSL, will be transformed into SP IK1MSL@IK1ZZZ.

TSTHOST.PSW - Automatic password file

This   file  contains the  password for  those systems that require

an access password, like BBS and node sysop.

The  next manual  section describes  this aspect with more details.

If  you do  not need  any password,  or do  not need  an  automatic

password,  do not  create this  file. Anu  line starting  with  the

'#' symbol is a remark and is ignored.

TSTHOST v1.42 (c) IK1GKJ                                     14

TSTHOST.SYS - Password file for PMS SYS command

This file is similar to the tsthost.psw file. The format is:

CALLSIGN_without_ssid VALIDATION_FLAG PASSWORD

every  line start  with   a callsign   without  SSID,  a  space,  a

validation   flag  and  the  user  password  not  longer  than  255

alphanumeric  characters. Only  the users write here may do the SYS

command. For instance:

IK1GKJ 3 aabbcc

IK1MSL 1 8873mnhbkjAST

Flag is a bit field:

1 means  that user  may access  with the YD, W, V to any drive

and path of your system, but may YU only in the UPLOAD path.

2 is like 1, but the user may access to any disk and path also

for write a file (YU)

4 means that the user may do the DOS command.

8 means  that the  user may  do yapp transfer even the yapp is

disabled in the pms. (command PYAPP)

16 means  that the  user may access to all message in the pms,

exatcly  like if  hi is the sysop on the keyboard. Access including

the editing of the mail header. (EM, FA, FR commands)

In  the example  above, IK1GKJ  may read  and write  in any part of

your  system (1+2=3), IK1MSL instead may read in all your disk, but

write only in the UPLOAD directory.

TSTHOST.CFG - Automatic configuration file

This   is   an   ASCII    text  file,  that  contains  the  command

sequence  for   the   automatic  configuration.  This  file will be

searched  by TSTHost  when   started and,  if present,  the program

executes  all the  commands   in the  file. Every   line    is    a

command.   This   file  can contain  any TSTHost command.  Any line

starting  with the '#' symbol is  a remark  and  will  be  ignored.

The  line  length  MUST  not exceed 78 characters.

TSTHOST.SHD - Shutdown configuration file

Like   the  tsthost.cfg   file,  this  file  contains  the  command

sequence that is executed at the program shutdown.

TSTHOST v1.42 (c) IK1GKJ                                     15

???????.ULS - Unproto list file

This   file  is created  and  managed  by  the  program.  When  you

activate  the   unproto   list  the  program  reads  this  file, if

present,  so TSTHost   can  handle the  message list  from the last

number  received. When   you   close   the  list  or  shutdown  the

system,  the program  saves the  content of  the list  in this disk

file.  If   this   file  is not found or you delete it, the program

will  ask to  the HomeBBS  the Unproto message starting from number

one.  The extension  filename is  always ".ULS", while the filename

is the callsign of homebbs without ssid.

TSTHOST.ENT - PMS connection text

This  is an   ASCII   text   file.  When a remote user connects the

PMS  system,   TSTHost sends  the content  of this  text  file  (if

found) to the user before the PMS command prompt.

WARNING:  Do not use the square brackets '[' and ']' in this file,

because they are used for SID forward protocol.

TSTHOST.HLP - PMS help file

This  is an   ASCII   text  file. When the remote user connected to

the  PMS issues the  HELP  command,  TSTHost  sends  the content of

this  file to  the user,  otherwise sends  the message 'NO HELP' if

the file is not present on the system.

TSTHOST.PGS - PMS PG server list

Another   ASCII file.  It can be created by the sysop according  to

his  requests, because  it is  related to  the PG  section. When  a

user  connected   to   the   PMS   issues   the  command PG without

arguments,  TSTHost sends the contents of this file to the user, or

the  message NO PG AVAILABLE if the file does not exists. This file

can be used to list and describe the available PG server programs.

TSTHOST.LOG - Connections log

This  file will  be created  and   managed   by  TSTHost  when  the

LOG  command  is  ON.  It  contains  the  list  of the  connections

with callsign, date and time.

SOURCE files

These  files are  similar   to   the  tsthost.cfg  file,  but  will

be  executed  only  when  you  type  the  command.  This is  useful

to  activate specific  configurations on  demand. Any line starting

with the '#' symbol is considered a remark and is ignored.

TSTHOST v1.42 (c) IK1GKJ                                     16

TSTHOST.CRN - Command file executed every hours

This  command file  is executed, if exist, every hours. The must be

put  in the  same directory  of tsthost.exe,  and must be edit in a

pure  ASCII form.  NOTE: any  command may  be put  into this  file,

because  tsthost do  not execute  any type  of control, but please,

put command that have a sense!

The format is simple:

- any line starting with a # is a comment and ignored.

-  any other line are treated like tsthost command, exactly like if

the  command was  entered from the keyboard, and are executed every

hours.

-  Exception are  the line starting with the * symbol. This type of

line  must have  the format:   * HOURS COMMAND and will be executed

only at the specified hours.

Here is an example:

#----INIT FILE--------

RUN AUTO7P INQUIRYFILE C:\TSTHOST\WORK\7PDATA.FWD

RUN AUTO7P DELETIME 20

* 18 PYAPP OFF

* 18 RING ON

* 21 PYAPP ON

* 23 RING OFF

#----END EXAMPLE-----

Line 1 and 2 are always executed every hours.

Line 3 and 4 are executed at 18

Line 5 at 21

Line 6 at 23

Line lenght must not be excedeed the 80 characters.

Hours are defined from 00 to 23.

TSTHOST v1.42 (c) IK1GKJ                                     17

TSTHOST.BID - Bid file

This  is a very important file, and must NOT be altered or deleted.

The  purpose of this file is to have a list of the received bid and

to  store the number of the last bid used by tsthost. But what is a

BID?  Simple, think  to a  bid like  a word  that unique identify a

message.  Every time a new message will be created, from tsthost or

other  bbs, a  unique bid  will be appended to the message, this to

avoid duplicate.

For  example, assume  that i forward the message XX to homebbs, the

message  have bid  YY. Next i use the command FA to force again the

forward  of this  message to  homebbs.  When  homebbs  receive  the

proposal,  it detect  that the  bid YY  was already  received,  and

refuse the message.

The  internal bid of tsthost are a sequential numbers and a string,

you callsign: 12345-IK1GKJ

(1->2->3->..65535->1->2 etc etc).

Now, if you delete or lose this file, you obtain 2 things:

1)  will be  lose the  list of  the received  bid, this is a little

problem,  at most,  if you  receive forward from more than one bbs,

is possible that you receive a duplicate message.

2)  tsthost must  restart the  bid sequence,  and this  is a  great

problem.  For example,  tsthost  have  created  and  forwarded  ten

message,  this messages  have bid  from 1 to 10. Now, if you delete

the  file, tsthost must restart the sequence from 1, but since your

homebbs  have already  received bid  1..10, it will refuse any your

message with bid number under 11.

To  avoid this  problem, you can use the command NBID, to assign an

high  value to  the bid  number.  This  is  a  emergency  solution,

because  assume that  you know what was the last bid number used by

tstshost.....  The best  thing is  to NOT  DELETE or  altered  this

file.  Pleas note  that the  NBID command  must be  used  ONLY  for

emergency situation, normally you MUST NOT usa this command.

When  tsthost start,  if the bid file do not exist, the program ask

for  its creation  (normally the  first time  you  start  tsthost).

Insert  Y to  create the  file, (and  if the  file was  lost do the

command NBID BID_NUMBER), or N to exist and provide a backup copy.

TSTHOST.BID  a routing file of 3001 record. Every record is long 13

byte,  and is  the bid  of the message (char[13]). The first record

is  an exception,  it  have  two  unsigned  integer  number  (ascii

format),  separated by  space; the  first is  the next internal bid

that  tsthost will  be use,  the  other  is  the  first  record  to

overwrite into the tsthost.bid file.

TSTHOST v1.42 (c) IK1GKJ                                     18

TSTHOST.IN - Mail import file

This   is   an  ASCII text file. The purpose is to allow SERVER and

PG  programs   to   add   messages   to   the    PMS.     The  file

presence  is tested    every  minute  and  processed,  when  found.

TSTHost  searches in  this   file a  sequence of SP or SB commands.

Note  that you  MUST use  the   correct syntax.   When processed, a

line  starting with SP or SB is searched, discarding the others. If

such  line is  found, the  next one is considered the message title

and  the others  are the  message text.  A line  starting with  /EX

ends the message.

Example:

SP IK1AAA @ IK1MSL < IK1BBB

message title

message line 1

message line 2

....

/EX

SB ALL @ EU

message title

message line 1

message line 2

....

/EX

The   TO  field   MUST be  specified, the  ROUTE (@)  and FROM  (<)

fields

are  optional.   If the  route field is specified, the message will

be

forwarded   to  the   HomeBBS. If  the FROM field is not specified,

the

default value of the PMS callsign will be used.

NOTE: The line length MUST NOT exceed 80 characters.

TSTHOST v1.42 (c) IK1GKJ                                     19

TSTHOST.DMS - PMS messages database

This   is  a   binary file,   and is  used by  TSTHost to  maintain

the  messages   list in   the  MAIL directory. Do not alterate this

file  if you do not know what you are doing!. The easiest method to

destroy all messages is to delete this file, so be careful  ......

This   is    the    tsthost.dms    file    structure,    for  those

programmers interested in writing server programs:

struct dirmes {

char type;                  /* type of message BPT */

char status;                /* status of message NFKYHX */

long number;                /* number of message */

int  da_year;               /* creation date, year */

char da_day;                /* day */

char da_mon;                /* mounth */

unsigned char ti_min;       /* creation time, minutes */

unsigned char ti_hour;      /* hour */

unsigned char ti_hund;      /* hundredths of seconds */

unsigned char ti_sec;       /* seconds */

char to[7];                 /* destination, TO field */

char route[7];              /* destination BBS, @ field */

char from[7];               /* sender, FROM field */

char title[61];             /* message title */

char frombbs;               /* bit field:

bit 0 =1 if received in forward, 0 if local

bit 6 =1 if the msg contain the string /ACK

bit 7 =1 if the msg contain 7plus code */

char startbbs[7];           /* BBS where the message started*/

char bid[13];               /* BID or MID of the message */

char rcvbbs[7];             /* BBS that have fwd the msg */

char free[4];               /* reserved */

};                 /* total 128 bytes */

All char fields end with a 0 byte, according to the C default.

The  message is  decoded with  the number  field. The message is in

the  MAIL   directory:   the   filename   correspond to  the NUMBER

field  and the  extension is  .MES. The  filename is always 8 chars

long,   right aligned by padding the remaining left positions  with

ASCII  '0's. For   instance,   if  NUMBER is  4,   the      message

filename   will  be 00000004.MES

The  message number  is consecutive   and,    when    the    server

appends  a new  message,   it   has   to  read the last record from

tsthost.dms  and increment  the number  field  to  obtain  the  new

message number.

WARNING:   all  fields in  the dirmes  structure must  be always be

set;  for instance,  if the  @ route is not requested, you MUST set

it, so *dirmes.route=0;

DO NOT ALTER THIS FILE WHILE TSTHOST IS RUNNING !

TSTHOST v1.42 (c) IK1GKJ                                     20

TSTHOST.EXT - File extension for automatic execution

----------------------------------------------------

When  you do  the DIR  command, the  contents of  the directory  is

displayed  on the  windows screen.  Now, if you press the ENTER key

on  a filename,  tsthost will  search a  corrispondance between the

EXTENSION  of the selected file and the contents of the tsthost.ext

file.  If the  corrispondance is not found, or the file tsthost.ext

does  not exist,  the pressure of the enter key is ignred. Instead,

if  the corrispondance  exist, tsthost,  (if you  have  the  needed

memory),  will execute  the specified  ds program  defined for that

corrispondance. You may execute, .bat, .com and .exe program.

The format of the tsthost.ext file is simple:

-Any line mut be long not more of 80 charactes. ONE line for ext.

-Any  valid line  start with a . followed by the extension and next

by the instruction to execute fot that extension.

There  are two  symbol that tsthost may substitute with the name of

the selected file.

1) the character "&"

For  the first & found, tsthost will substitute the symbol with the

full  path and  filename for the selected file. The related command

is called from the tsthost directory.

2) the character "!"

For  the first ! found, tsthost will subsitute the symbol with ONLY

the  filename of  the selected  file, next change to drive and path

of the selected file, and executed the related command.

Example:  current displayed  directory is  D:\BIN, and the selected

file is MARCO.ARJ

1) assuming to have a line like this in tsthost.ext:

.ARJ C:\UTILITY\AVIEW & /K /J

When you press return, tsthost will execute:

RUN C:\UTILITY\AVIEW D:\BIN\MARCO.ARJ /K /J

2) assuming you have a line like this in tsthost.ext file:

.ARJ C:\UTILITY\AVIEW ! /K /J

When you press return, tsthost will execute:

D:

CD \BIN

RUN C:\UTILITY\AVIEW MARCO.ARJ /K /J

TSTHOST v1.42 (c) IK1GKJ                                     21

TSTHOST.MAC AND THE MACRO - File definition for macro command

-------------------------------------------------------------

What is a macro?

----------------

First,  what is  macro? A  macro is  a  sequence  of  command  that

tsthost  will execute automatically, sequentially, exactly like you

type  the same  commands from the keyboard. The command may be data

to  send to  the remote  station, or  command for tsthost, or macro

relative  command. It is very important understantd the the command

will  be executed  sequentially:  this  means  that  we  need  some

interrupt  point to  stop the  sequence, otherwise  the macro  will

nevar stop and continue to its end without do the request result.

Again,  we are  intelligent, but  the  macro  no,  macro  are  very

stupid, so you must instruct tsthost exactly what to do.

How  start a  macro? Simple  typing its  name in  the cmd  mode  of

tsthost.  Every time you type something that is not recognized like

a  tsthost command, the program will search in the tsthost.mac file

if  exist a macro with that name. If the macro do not exist, or the

file  tsthost.exe do  not exist,  the command  is  simple  declared

invalid.  If the  macro  exist,  the  macro  will  be  executed.  A

flashing  "M" will  appear on  the status  line to  display that  a

macro is active on that channel.

How  to stop  a macro?  The command is MABORT, and have a immediate

effect:  macro will be stopped, and the channel remain in the state

that was at the moment in which the macro was stopped.

The  macro support  all tsthost  command, with  a little exception,

and  have a  subset of  internal commands,  valid  only  for  macro

execution.  Is possible  insert also  a name  of another macro like

command,  but in  this case  the new macro replace the existing one

in the execution.

NOTE:

Any  line, in  the macro  structure, that  is preceeded  by + sign,

will  be executed  ONLY if  the channel  is connected. For example,

the  statement +SAY  ...., will  be  executed  if  the  channel  is

connected,  skipped otherwise.  SAY ....  instaed, will  be  always

executed. Do not put space between the + sign and the command.

TSTHOST v1.42 (c) IK1GKJ                                     22

Macro specified command

-----------------------

NOTE:  the command may be written in upper case or lower case. Some

of  those command  will refer to a line number: Yes, the macro have

the  line number,  like the basic language, but this number will be

explaned   next  in   the  relative   section  of  the  tsthost.mac

structure.  For the  moment, you need to know that if you refear to

a  line number  that do not exist in your macro, the macro will not

produce  an error,  but will go to the existent numeber immediately

highest  of the  number that  you have  specified. This  means  two

things:  if the  inexisted number  is lower  of the lowest existent

number,  the macro  will be  restarted. If the inexistant number is

higher  than the highest existent number the macro will be stopped,

beacuse you reach the end of macro.

* WAITDISC [XX]

This  statement simple  wait until  the channel is disconnected. If

the  chanel is  connected, no  other macro  line will  be processed

until  the channel  will  be  free.  If  anything  follow  waidisc,

example  WAITDISC  X,  the  program  wait  until  ALL  channel  are

disconnected

* DELAY XX

XX  is a  number to  specify how  second the macro must wait before

process  the next  instruction. This  is a REALLY important command

in  the connection  statement. For  example, assuming you connect a

station,  and open  a record  file. The  record command  will fail,

because  you have  send the  command CONNECT,  but immediately open

the  record file.  In this  very little time, probably the tnc does

not  have tell  tsthost that  the channel  is conected,  so tsthost

will refuse to open the file.

ALWAYS ADD A LITTLE DELAY AFTER A CONNECT STATEMENT.

* IFYAPPACTIVE XX

XX  is the  line number where tsthost will go if, when process this

line, there is a yapp transfer active on the macro channel.

* IFNOTYAPPACTIVE XX

XX  is the  line number where tsthost will go if, when process this

line, there is not a yapp transfer active on the macro channel.

TSTHOST v1.42 (c) IK1GKJ                                     23

* IFYAPPERROR XX

XX  is the  linenumber where tsthost will go if the latest executed

yapp  was stopped due to an error. NOTE: reading this variable will

clear  the yapp  error status,  so you  cannot use  it two  or more

times.  After reading  the error status, you may read it again only

after a new yapp transfer.

* IFNOTYAPPERROR XX

XX  is the  linenumber where tsthost will go if the latest executed

yapp  was succesfully  terminated. NOTE: reading this variable will

clear  the yapp  error status,  so you  cannot use  it two  or more

times.  After reading  the error status, you may read it again only

after a new yapp transfer.

* IFFILESEND XX

XX  is the  linenumber where  tsthost will go if, when process this

line, there is a file transfer active (SEND command).

* IFNOTFILESEND XX

XX  is the  linenumber where  tsthost will go if, when process this

line, there is not a file transfer active (SEND command).

* GOTO XX

XX simple is the line number where tsthost will go.

* SAY DATA DATA DATA......

This  statem,ent tell tsthost to send DATA.. to the remote station,

is  like when  you are connected and type something to the keyboard

in  the CONV mode. DATA will be send to the remote station. You may

also  use the  tsthost SEND command to send long data to the remote

station.

* WAITSTRING TIME (LINENUMBER) STRING TO WAIT

This   statement  is   more  complex.  Tsthost  when  process  this

instruction  wait for  not more  of TIME  seconds  to  receive  the

"STRING  TO WAIT"  from the  tnc.  If  "STRING  TO  WAIT"  was  not

received  in TIME  seconds, the cprogram will go to the line number

specified  in LINENUMBER. All field are needed in this instruction,

and () are mandatory aroud the linenumber value.

TSTHOST v1.42 (c) IK1GKJ                                     24

TstHost.mac file format

-----------------------

Another  simple file  in the  format, more  complex is  write macro

that work right hi!

- Any line MUST NOT have more of 80 characters.

- Any line that start with a # characters, and any empty line is

ignored.

A  macro start  with its  name. The  macro may  be executed  on the

cirrent  channel, or  may have  a specified channel: this is needed

on  a multiport  system, where a call, for example, may reched only

trought  a specified  port. If  i hae  two radio port in my system,

and  my bbs  frequency is associated on the channel 5,6,7,8m surely

i cannot connect it on the channel 3, no?

A  macro stop  when a  new macro  start or when the end of file was

reached.

To  define a macro name, the line must start with the ":" character

followed  by the  macro name.  Macro name  may have  not more of 20

characters,  and may  be followed  by a  number  to  specify  which

channel  will be  used for that macro. Obviusly, the channel number

must not be highest of the number of channel defined for tsthost.

Here is two example:

:PIPPO

This macro will be executed on the actually current channel.

:GIOVANNI 5

This  macro will  be executed on the channel number 5. You may also

use  channel 0,  monitor, for a macro, but remember that you cannot

do connection on the monitor channel.

Any  other line  in the  macro definition start with a line number,

that  is a  sort of  label, followed by a space, and by the command

to  execute. NOTE:  line number  must be  in sequential  up  order,

tsthost do not do a sort order for the macro line number.

Ogni  altra riga  della macro  inizia con  un numero  di riga,  che

particamente  e' una etichetta, seguito da uno spazio e dai comandi

associati  a quel  numero di  riga. Attenzione,  non potete mettere

numeri  di riga  fuori sequenza.  I numeri  devono essere in ordine

crescente, tsthost non effettua l'ordinamento.

For example:   right               bad

10 PIPPO            20 PIPPO

20 LUPO             18 LUPO

TSTHOST v1.42 (c) IK1GKJ                                     25

You  may use  line number from 1 to 32767. 0 cannot be used because

is  an internal  number, and from 32768 will be considered negative

number from tsthost and are invalid in the macro.

Pratically, a macro may have an aspect like this:

:NAME [OPTIONAL CHANNEL]

10 INSTRUCTION ...

20 INSTRUCTION ....

...

.... ETC.

TSTHOST commands not active in the macro

----------------------------------------

This commands are not available in the macro:

DIR,  VIEW, ULIST  LIST, ULIST  VIEW,  because  open  a  window  in

tsthost, and require an active presence on the tsthost keyboard.

SOURCE,  because  i  cannot  do  a  preliminary  control  over  the

contents of a command file.

TSTHOST commands that require an detailed arguments

---------------------------------------------------

This  commands, may  be used in a macro structure, but require that

the arguments is specified in the command line.

DELETE,  SEND, YPUT,  because if you do not do the arguments open a

windows that permits the directory selection.

RUN,  because if  executed without  parameters shell to dos without

reenter  in tsthost  until the  operator do  not type  EXIT to exit

from the shell.

TSTHOST v1.42 (c) IK1GKJ                                     26

Macro example

-------------

:PIPPO 2

10 CONNECT IK1MSL-8

20 DELAY 2

25 +RECORD POSTA

30 +SAY RM

35 +SAY KM

40 +SAY B

This  is a  really simple  macro. It  connect my  bbs on channel 2,

note  the delay  instruction please.  Next opne  a record file, and

queue  in the bbs the needed command to read and kill my mail, next

disconnect.  All the macro, are executed in a unique time. When the

macro  stop, probably  the connection  is again active, but all the

needed  bbs command  was done.  When the  bbs  disconnect,  tsthost

automatically  close the  POSTA file, and nexty i can examine it to

read  my mail.  Note again  that line 25..40 are executed only if a

connection was done, due to the + sign before the commands.

Here  is a  more complex  macro. In  this macro,  i assume that the

prompt  of my  bbs is  "IK1MSL BBS  =>". Again,  my bbs  ask for  a

password with the password prompt "PASSWORD?".

>>For the password please read the relative section in the doc<<

:MSL 3

5  WAITDISC

10 CONNECT IK1MSL-8

12 DELAY 2

14 +WAITSTRING 30 (1000) PASSWORD?

16 +PASSWORD

18 +WAITSTRING 30 (1000) IK1MSL BBS =>

#

40 +YD VARIE\PIPPO.ARJ

42 +WAITSTRING 30 (100) IK1MSL BBS =>

44 +IFYAPPERROR 40

46 +GOTO 1000

#

100 +IFYAPPACTIVE 42

110 +SAY

120 +WAITSTRING 30 (1000) IK1MSL BBS =>

130 +GOTO 44

#

1000 DISC

TSTHOST v1.42 (c) IK1GKJ                                     27

005: wait until the channel is free.

010:  connect my  bbs using channel number 3. Again note the little

delay  to recognize the connection. All the other line will be

executed  only if  connected, due  the +  sign on the start of

the command.

014:  wait for  not more of 30 seconds the password request. If not

received,  sonthing is wrong. Bbs may be crashed or may be out

of  program due  to dos  operation. In  this case i will go to

line 100, where force a disconnection and stop the macro.

016: send my password.

018:  and wait  for the  bbs prompt.  Again, max  time wait  is  30

seconds,  if  in  this  time  the  prompt  was  not  received,

assuming  that something  is wrong,  and go  to line  1000 for

disconnect and stop.

040:  ask to  download the  file PIPPO.ARJ,  that are in the VARIE\

directory, using yapp protocol.

042:  wait for  not more  of 30  seconds to  obtain the bbs prompt,

that  signal the  and of  the yapp  transfer. 30  is a  little

number but do not have now a great importance.

044:  when the prompt is received, if i have an error ask again the

file going to the line 40.

046: no error, so go to line 1000, disconnect and stop macro.

100:  due to the statement in line 42, if after 30 seconds i do not

receive  the bbs  prompt, tstshost macro will jump here. First

test  if the  yapp transfer is again active. If is active, ask

for more time jumping again in line 42.

110:  if the transfer is stopped, but i have not received a prompt,

is  possible that i have an abirt yapp. fbb 515c have a ad bug

in  the abotrt  yapp routine:  it stop to send data, but do ot

confirm  the abort  to the  corrispondance  with  the  correct

sequence.  Due to  this, 90%  of case  the remote station wait

for  an abort  ack from  fbb while  fbb have  already stop the

yapp   transfer.  To   avoid  this,   and   resyncronize   the

communication  with the  bbs, i  will send  an empty  line, to

force the bbs to emit its prompt.

120:  wait here  the prompt.  Again, if not received, disconnect on

1000.  May be  that there  was not  an error  in the  yapp, is

possible that the bbs is crash.

130:  if receive  the prompt,  jump on  line 44  to test  the  yapp

status.

This  macro is not yet perfect. The problem is that if you continue

to  receive  error  on  the  yapp  transfer,  the  macro  will

continue  to ask  for the  file. But  this is only an example,

you may surely study a macro without this problem.

Obviusly,  macro, prompt,  timing, password  are only  example, you

must insert the appropriate value for you needed.

TSTHOST v1.42 (c) IK1GKJ                                     28

AUTOMATIC PASSWORD

------------------

The  TSTHOST.PSW  file  contains  the  passwords to use with those

systems where it is requested for the access. The file format is:

CALL STRING_TO_SEARCH PASSWORD

CALL is the callsign for the referred password. WHITOUT SSID

STRING_TO_SEARCH   is   the   system  prompt when  the password  is

being  requested, max  50  character long  with no spaces, PASSWORD

is  your  password,  up  to  255  alphanumeric  characters  without

spaces.

Example:

IK1MSL IK1MSL-8> abcdef

IK1MSL Password? 123456

IK1GKJ IK1GKJ-8> ABCEDF

Warning:  Upper and  lower case are ignored  in  the  string being

searched, but are important in the password string.

Here  are some  example for  the BBS  prompt, that  can  solve  any

doubt,  and make you write the correct tsthost.psw. In the example,

my demo password is ABCDEF

IK1MSL-8> 11 22 33 44 55 [0123....           IK1MSL-8> ABCDEF

Password? IK1MSL-8> 11 22 33 ....            IK1MSL-8> ABCDEF

IK1MSL-8> Password? 11 22 33 .....           Password? ABCDEF

IK1MSL-8> Password ? 11 22 33 ....           ? ABCDEF

As  you have  seen, the  string being  searched is  ALWAYS the LAST

word or characters that came before the password code.

If the BBS prompt is:

Insert your password 11 22 33 44 55

and you insert in your tsthost.psw file:

Insert your password ABCDEF

TSTHost  will interpret  "Insert" as the string to be matched, and

"your"  as your  password. This  is an   error! The correct form in

tsthost.psw must be:

password ABCDEF

TSTHOST v1.42 (c) IK1GKJ                                     29

NOTE:  if you  insert like  call an  '*', the program will not test

the  equivalence between  the line and the connected call, but only

the  equivalence between  the prompt received and the prompt in the

line  of the  file. So if you use this capability, make sure to put

the * lines at teh end of the file, to avoid confusion.

IMPORTANT, PLEASE READ

----------------------

This  information are  not valid  for the  automatic connection for

forward  and unproto  mail request,  since they  have a particoular

protocol.

When  you do the PASsword command, or press the ALT-F1 key, tsthost

will  start the  serching of  the password  with the comparation of

the task callsign and the callsign declared in tsthost.psw

If  you do  direct connection,  this work  without any problem, but

look  for this  example. I  connect a  node IK1GKJ-7. Ok, IK1GKJ is

the  task callsign.  Now i send to the node, the command to connect

my  bbs, IK1MSL-8,  and the  node connect my station with IK1MSL-8.

Now  IK1MSL-8 will  ask for  the password,  i press  ALT-F1 or type

PASsword command, and tsthost emit an error message!

Why?  Because for  the tnc  you  are  connected  to  IK1GKJ-7,  not

IK1MSL-8,  since the  link to  ik1msl was done in the NODE, and NOT

in your TNC link.

So  tsthost will  search an abbination for the password prompt with

IK1GKJ  and not  IK1MSL, and  since do  not found  it,  declare  an

error.  To avoid this, PASSWORD command accept a parameters; if you

do the command:

PASSWORD IK1MSL

Tsthost  will ignore  the connected  callsign and  execute a search

for  IK1MSL callsign,  ignoring the  task  call,  IK1GKJ  in  thist

example.  For frequently connection with password trought node, you

may  find useful to program the related PASSWORD CALLSIGN in one of

the 20 programmable function key, to emulate the ALT-F1 short cut.

TSTHOST v1.42 (c) IK1GKJ                                     30

FORWARD,  UNPROTO  list,  CHANNEL  8  and automatic password

------------------------------------------------------------

FIRST  READ THIS:  In this  section, channel 8 is to be intended if

the  program is  setup for  all 8  channel. If  you have change the

number  of available  channel with  the /K  command,  the  forward-

unproto  request will  be done  on the  last available channel. For

example,  if you ha start tsth with the command /K4, channel 8 here

is to be intended like channel 4.

TSTHost  connects the  HomeBBS  for  forward  or  unproto  list  on

channel  8 only.  If this  channel is  BUSY, the  program waits and

retries   to connect the station when this channel is free. For the

forward,  the program  always calls  the master  BBS to  allow  the

reverse  forwarding but  the unproto  list  is  requested  only  if

active and there are some selected messages.

NOTE:  in a  multiport system  (multiport TFPCX),  you MUST  ensure

that the channel 8 is associated to the same port of the HomeBBS.

Again,  since it  is possible  that the program  calls  the station

while you are already connected to the HomeBBS on another channel,

it is suggested to set for channel8 a  unique  SSID (AX25 PORTCALL

command).

???????.CON - HomeBbs connection file

-------------------------------------

This  file permit  to define  exactly how to connect you homebbs in

order  to ask  for unproto  messages or  do forward  procedure. You

need  a file  for any  homebbs used,  the name  of the  file is the

callsign  of the  bbs without  ssid, the  exetnsion is always .CON,

and  the file  must reside  in the  directory where is tsthost.exe.

For  example, my  homebbs is  IK1MSL-8, so  i have  a  file  called

IK1MSL.CON

The file have this rules:

- Any line must be at most 80 characters lenght.

- Any empty line, and any line that start with # is ignored.

- Any other line must start with a valid command.

Actually, four are the supported command:

-  C, tell  to tsthost  that this  is a connection line. The format

is:  C (CALL-SSID)  CONNECTION COMMAND.  The () are needed, and the

callsign  inside is  the callsign  of  the  connection.  CONNECTION

TSTHOST v1.42 (c) IK1GKJ                                     31

COMMAND  is the  command  to  perform  in  order  to  connect  that

station.

-  S, change  the answer  return prompt  on the connection command.

This  is normally used only for that node that do not have standard

return  messages. Normally  all node  return CONNECTED  in case  of

right   connection,  or  FAILURE  or  BUSY  in  case  of  a  failed

connection.   This  messages  are  already  defined  internally  to

tsthost,  and do  not need  to be  redefined.  But  some  node  may

return,  for example,  CONNE instead of CONNECTED, ECHEC instead of

FAILURE, OCCUP instead of BUSY. The command:

S CONNE ECHEC OCCUP

advise  tsthost that  from this  point all  the connection messages

will  have that messages. The redefinition will remain active until

a new redefinition will be made.

ALL 3 PARAMETERS MUST BE SPECIFIED.

-  L COMMAND  VALUE, This  command is  used  to  setup  particoular

parameters  in the TNC. All data after the L command is sent to the

tnc,  like command.  The command  must have the WA8DED host sintax,

parameters  modified have  effect only  on the  connection channel,

and  will be  restored to  the orginal  value at  the  end  of  the

connection.

-  $ PASSWORD  PROMPT. If  you bbs require a password to access it,

you  may define  here the  password request  prompt.  This  is  the

oldest   HPASPROMPT  command.  For  more  details  about  automatic

password, and configuration file, see the related section.

Here  is some  example, referred  to my homebbs IK1MSL-8, so to the

file IK1MSL.CON:

#bbs may be reached directy, so

C (IK1MSL-8) C IK1MSL-8

|    |       |

|    |       |--from here to end the connection command.

|    |----------callsign of the connection

|---------------tsthost command

TSTHOST v1.42 (c) IK1GKJ                                     32

#bbs is reached directly trough digipeaters

C (IK1MSL-8) C IK1MSL-8 VIA IK1ABC IK1QWE

#More complex, bbs need node and digipeaters to be reached.

#First, i need to connect the node IK1AAA-7

#

C (IK1AAA-7) C IK1AAA-7

#

#now, i need to connect the node IK1PIP-2, but here i have some

#problem. First, the node AAA-7 do not use the C command to

#connect, but use X. Next, the return message for busy is not BUSY

#but OCCUP, the other messages are standard.

#

S CONNECTED FAILURE OCCUP

C (IK1PIP-2) X IK1PIP-2

#

#Ok, now i may connect my homebbs IK1MSL-8, but warning. PIP use

#standard connection messages, so i must first redefine the

#previously altered messages. PIP do not heard directly ik1msl-8

#so i need to use the digipeaters IK2DED

#

S CONNECTED FAILURE BUSY

C (IK1MSL-8) C IK1MSL-8 VIA IK2DED

#This bbs may be reached directly, but require a password to

#access it. Password prompt is PASSWORD?

#I will use maxframe 3 for this connections.

#

L O 3

$ PASSWORD?

C (IK1MSL-8) C IK1MSL-8

TSTHOST v1.42 (c) IK1GKJ                                     33

FORWARD protocol

---------------

The   PMS  can forward  with a master BBS according to the standard

MBL/RLI  or FBB  COMPRESSED protocol.  Any BBS  may connect the PMS

and  forward data,  but the   PMS  will  send  the  message only to

the BBS defined with the HOMEBBS command.

The   PMS   will  send  to  the HomeBBS  all the messages that have

a  specified   route set  ( @Field not blank), and locally created.

If  a received   message   has  been forwarded  by another  BBS, it

will   be forwarded to the  connected  BBS  only  if  the  callsign

of   the  connected   BBS is not the same of the callsign stored in

the RCVBBS field.

For  personal  messages,  they  will  be  forwarded  only  if  the

addressee of the message has not read it.

In   the message list, the letter 'L'  near the type/status of  the

message  shows that   this  message was  written locally, otherwise

the message was received by the forwarding activity.

You  may always  connect Home  Bbs and  send your mail, but to have

forward,  you must be enabled from the sysop of the bbs. So contact

the sysop and ask for this feature, if you are interesting.

To do this, the sysop of Home Bbs must:

1) Create a forward file like this:

#

A IK1GKJ

P B

R

B IK1GKJ

F IK1GKJ

C C IK1GKJ

----------

#

If  you also need  the  @AMSAT  messages, the  sysop  must  add

the G AMSAT statement after the F IK1GKJ line

2) Edit the BBS.SYS file, and add your CALLSIGN to the list

3)  Edit your  record and  change the bbs field, now it must be set

not to bbs callsign, but to YOUR callsign. (NH command.)

This  info is  for fbb  type bbs,  i do not know how work the other

sotware.

TSTHOST v1.42 (c) IK1GKJ                                     34

Problem to connect for UNPROTO and FORWARD

-----------------------------------------

TstHost  use to  call homebbs  for fwd  or unproto message a simple

protocol.  First, will  be connected the bbs, always on the channel

8,  using HNode  if set,  or  directly.  Next  TstHost  wait  until

receive  the from  the BBS,  the SID  and/or PASSWORD  request, the

order  is not  rilevant. But  the SID must be received, (sid is the

string  [FBB-5.15-ABFHM$] or  similar). The  program  MUST  receive

this  string, because  from the sid the program can detect the type

of protocol to use.

Now,  tsthost wait  for a  bbs  prompt.  When  the  bbs  prompt  is

received,  the sequence  is completed,  and forward or unproto list

request may started.

The  bbs prompt,  is a  char >  at the end of a line. This may do a

problem  on some  bbs, since  many sysops  have set the char > into

the  welcome text  on their  bbs. In  the normal connection this is

not  a problem,  but with a remote protocol yes, because if tsthost

receive  this character,  it think  that it  is the bbs prompt, and

the sequence was not corrected.

For example, if the bbs welcome text have a line line this:

HALLO MARIO, FOR HELP TYPE THE COMMAND <?>

surely  you will  not be  able to start fwd or unproto, because the

character > at the end of the line confuse tsthost.

But,  this line,  HALLO MARIO, TYPE <?> TO HELP, do not do any type

of problem.

If  you have  this problem, you may ask to the sysop bbs to set you

like  a real  bbs. The  bbs nevar  send text  to real bbs, but when

connected send only the prompt (or password request).

NOTE:  If you cannot start fwd o unproto list, and cannot solve the

problem,  to  obtain  help,  please  switch  on  the  fwd  channel,

manually  start a fwd or unproto request, and when disconnected, do

the  command WRITEBUF ABC. Next send me a message whit the contents

of the ABC file, so i can examine it.

Another  common problem, using FBB compressed mode forward only, is

that  when your homebbs must send mail to you, its send FF and mark

the  message like  forwarded. This is a bug into fbb, to solve, you

must  connect your  homebbs, and  set  your  default  bbs  to  your

callsign, with the command NH YOURCALL

TSTHOST v1.42 (c) IK1GKJ                                     35

PMS INTERFACE

-------------

Every  channel of  the software is a PMS and  a  normal connection

channel at the same time. All non-connected  channels  are  set by

default in PMS mode. This means that a remote  user  that connects

your station will operate in PMS mode.

But,  if YOU  issue the  Connect command,  the  channel  becomes  a

normal conversation channel.

There  is   an exception  to this. When the software starts, if one

or  more   channel in   your  TNC   are already  connected for  any

reasons,  they are  conversation channels,  not PMS.  The reason is

that   it   is  impossible to  know if  those  channels  have  been

connected by  us  or by a remote user.

You   can  switch   from PMS   or  NORMAL  channel  mode  with  the

commands  TALK   (from PMS to NORMAL, for example after a chat with

the  remote user),   and   PMS   (from  NORMAL to  PMS). You cannot

switch  a session  if the  task is  not stand-by. The PMS cannot be

executed  if there is an active file transfer on the channel (YAPP,

send  or capture). A  TALK  command  has  no  effect  if the PMS on

that channel is processing a user command.

Moreover,  you cannot switch a window to conversation mode  that is

already  active in  PMS mode.  This is  correct because  you cannot

chat  with a  user, or  your  text  will  interfere  with  his  PMS

activity. You have to switch to TALK mode before chatting.

MAIL WORD FLAH IN SCREEN

------------------------

When  a new mail for you arrive on the pms interface, the word MAIL

will flash in the screen.

To  stop flashing,  you MUST press ALT-F10 key or do the LM command

from KEYBOARD; i have used this soluction for two reasons:

1)  TSTHOST.DMS is  not in  memory, and scan it to see if your mail

is already read cost some time.

2)  a PIRATE  user may  connect your  pms with  your callsign, read

your  mail, and  next kill  this message.  I case  1) the line will

flash off, and you do not see that new mail was arrived. With this

system,  you may  note that  mail is arrived, even if that mail was

killed or read.

TSTHOST v1.42 (c) IK1GKJ                                     36

PRIVATE DIRECTORY FOR PMS USERS

-------------------------------

For  any user  connected to  the PMS interface is possible define a

private  directiry with the utility program MAINTUSR, provided with

TstHost.  The user  that connect  your pms  if have  defined a PRIV

directory  will may  access it with the virtual drive prefix P: The

user  will access  to any  subdirectory of  the priv, but not to an

eventually upper directory. For example, assuming exist this tree:

C:\TSTDATA\PRIV\LEVEL1\LEVEL2

Is   defined  like   priv  dir   C:\TSTDATA\PRIV  for  IK1GKJ,  and

C:\TSTDATA\PRIV\LEVEL1 for IK1MSL.

With   the  prefix   P:  (es   YD  P:xxx)  IK1GKJ  will  access  to

C:\TSTDATA\PRIV, LEVEL1 and LEVEL2.

while IK1MSL will access only to C:\TSTDATA\PRIV\LEVEL1, LEVEL2.

NOTE:  Even is  possible (BUT  NOT RECOMENDED) to use MAINTUSR with

tsthost  active, if you change a path for an user already connected

to  the pms,  the new  path will  not became  active until the user

disconnect and reconnect.

TSTHOST v1.42 (c) IK1GKJ                                     37

THE SERVER

----------

What  is the  server? The  server are programs com or exe that must

be  exist in  the tsthost.exe  directory. When  a personal  message

arrive  to your  pms interface,  and tsthost  detect a  server (see

SERVER  command), tsthost  copy the  contents of  this message to a

file,  next execute  the related  com or  exe file  and  pass  like

argument  the name  of this  file. This is a standard forward file,

so  the first  line will  contain SP SERVERNAME < IK1AAA, next line

will  be the title of the messages, next to end the contents of the

messages.  Server may  do anything, but any return message from the

server  to sender must go into the tsthost.in file. When the server

stop  the execution,  control return  to tsthost,  messaged  copied

will  be deleted,  original message  will  be  put  into  K  state.

Remember that will server are running, tsthost is stopped.

PG SERVER

---------

The  PG servers  are programs  written to  expand the capability of

the  software. They  MUST be  placed in the directory PG, one level

below  the  location  of  tsthost.exe. As  the  system  is inactive

while  executing   the  PG  servers, they must be small and use the

minimum  machine time. The displayed  image  is  saved  during  the

program  execution. The  server  must  NOT  access  the  video  RAM

directly.

The server receives the following command-line:

argv[0]     Drive\path\servername;

argv[1]     Callsign of the user, with ssid

argv[2]     Server level, 0 the first call, up to 255 the next.

argv[3...]  User-specified parameters, each word  is  a  separated

argument.

The  server data  output (with  printf() and putf()) is transmitted

to the user.

The server may return:

0 - The output of the server is  sent  to  the  user  and  returns

to the PMS prompt

1 - The output of the server is sent to the user and the server is

called again. Level incremented

2 - The output of the server is executed like a PMS  command,  and

the user is reconnected to the PMS interface.

TSTHOST v1.42 (c) IK1GKJ                                     38

3 - The output of the server is executed like a PMS  command,  and

the server is called again. Level incremented.

4 - The output of the server is sent to the user and the server is

called again. Level NOT incremented.

128..255 - The output of the server is sent to the user and the

server is called again. The level will be set equal to the PG

return code

NOTE   for  2   and 3:   The  output must be maximum 80 chars long,

and  must start  at the  line beginning;   the  line must NOT start

with   a  carriage   return or  line feed.  The server  must  NEVER

return another PG command.

For   instance, this is a very simple ECHO server  that  returns to

the user all he have typed, until the user types BYE:

#include <stdio.h>

#include <string.h>

main(int argc, char **argv)

{

int i,mode;

sscanf(argv[2],"%d",&mode);

if(!stricmp(argv[3],"BYE") && mode)

{

printf("\nBest 73\n");

return 0;

}

printf("\nHallo %s, you have send:\n",argv[1]);

for(i=3;i<argc;i++) printf("%s ",argv[i]);

printf("\nType BYE to end.\n");

return 1;

}

The user IK1AAA calls the server with PG ECHO AA BB.

The server returns HALLO IK1AAA, YOU HAVE SEND AA BB,

TYPE BYE TO END

the user types I AM MARIO

The server returns HALLO IK1AAA, YOU HAVE SEND I AM MARIO,

TYPE BYE TO END

the user types BYE

The   server returns BEST 73,  and the  user will return to the PMS

prompt.

TSTHOST v1.42 (c) IK1GKJ                                     39

CON_FILT.EXE, connection filter

--------------------------------

CON_FILT.EXE  is a  program that  may be  exist in  the tsthost.exe

directory. TstHost test if this program exist only when statup.

If  TstHost have  found at the startup time this program, for every

incoming  connections  on  the  pms  interface,  con_filt  will  be

called.  The purpose  of  this  program  is  to  provide  a  filter

connections,  but it  may do  anything you  need, you  may write  a

con_filt.exe program for any your need.

When TstHost call con_filt.exe, pass those arguments:

CALL-SSID of the user

Level, 0 the first time, up to 255 the other time

0 dummy, always 0

not 0 if is a new user, not registered in tsthost.usr

0 dummy, always 0

channel number where is connected the user

anything received from the user, each word a new argument.

Con_filt.exe must be return to TstHost:

0   to accept the connection

1   con_filt will be called again with the data received from the

user

2   connection refuse, the user will be disconnected.

Anything   write  from  con_filt  on  the  standard  ouput  channel

(printf(), puts()), will be send to the connected user.

TSTHOST v1.42 (c) IK1GKJ                                     40

MESSAGGE FLAGS

--------------

This  is a  little description of the letters that specify the type

and the status of the messages:

1 LPND IK1GKJ .......

||||

||||--Message contain 7plus code

|||---Messagge status (NYKFH)

||----Messagge type (BPT)

|-----Local

LOCAL:

The  L letter,  if present,  means  that  the  message  is  locally

created  on this pms. If absent, means that the message was receved

in forward from another bbs.

TYPE:

P  means that  the message is personal: this type of message may be

accessed  only from  the sender  and the  addressee of the message,

and only them may kill the message.

B  means that the message is a bulletin: Any callsign may read this

message,  but since  this message  is public,  only the  sender may

cancel it.

T  means Traffic,  i do  not have any info about this type of mail,

so it will be treated like the B type

STATUS:

N  means new mail. The message status may have some alterations: fr

example,  a personal  mail, if  readed from  the addressee, go into

the  Y status. Bulletin messages instead, do nt change to status Y,

because do not have a really addressee, but if for ALL.

X means that the message was rejectd from HomeBbs while forward.

F means that the message was forwarded to homebbs.

K means that the message was killed. A message killed is not

accessible from the user.

H means that the message is locked. A locked message is not

accessible from the user and will not forwarded until the sysop

will unlock it.

NOTE: The sysop may access to all messages.

TSTHOST v1.42 (c) IK1GKJ                                     41

PMS REMOTE MESSAGE MANAGEMENT

-----------------------------

Any  user, after  a succesfully  SYS  command,  have  the  complete

access  to all  message in  the pms if his sys flag have a value of

16 in the sum of his permission.

This  permission, enable  every  operation  on  the  pms  messages,

exactly like the sysop on the keyboard.

FA NSGNBR   Add the MSGNBR to the forward queue.

FR MSGNBR   Remove MSGNBR from the forward queue.

EM MSGNBR   Edit the header of MSGNBR.

THE TZ ENVIROMENT VARIABLE

--------------------------

Some  pms function, like rline, must convert your local time to the

GMT  Universal Time.  The  program  provide  automatically  to  the

conversion,  but for  a corrected conversion, your PC must have set

the TZ enviroment variable.

This  variable, TZ  (timezone), is  a standard  name  used  by  all

program  that make a conversion timing. The variable must be set in

your AUTOEXEC.BAT file, with the command:

SET TZ=XXXY

Where  XXX is  a 3  code letters, your time zone name, this data is

not  important for  the soft,  and  Y  is  a  number,  positive  or

negative,  indicating the  diference, in  hours, from  UTC time and

YOUR local time.

For  example, for  Italy, when are the 12.00 (solar time), UTC time

is 11.00. So i have put into my autoexec.bat file:

SET TZ=EST-1

TSTHOST v1.42 (c) IK1GKJ                                     42

PMS MESSAGE MANAGEMENT

----------------------

All  the manipolation  of the pms messages is done via a menu style

selection  in a  window. The activation of this window is done with

the L command, for LIST.

The messages list may be global or partial under filtering fields.

Those are the available commands for list the messages:

L              List all messages  (only LIST, not READ/KILL)

LL 50          List last 50 messages (only LIST, not READ/KILL)

L 100          List message number 100

L 100 102 150  List messages 100 102 and 150 (max 10 numbers)

L 100-150      List message from 100 to 150

L -150         List message from begin to 150

L 150-         List message from 150 to end

L< CALL        List all messages sent by CALL

L> CALL        List all messages for CALL

L@ ROUTE       List all messages for that route.

LS STRING      List all messages that have STRING in the title.

LM             List all my messages

LN (FKHYX)     List all messages in status N,F,K,H,Y,X

LQ             List all messages not yet forwarded.

LP             List all P messages

LB             List all B messages

LT             List all T messages

To  activate the  message window,  do the appropriate list command.

The  window will display the related messages, next you may move in

the  window withthe  classic cursor  movement keys: UP, DOWN, HOME,

END, PAGE UP, PAGE DOWN. The ESCAPE key will end the session.

In  list mode,  you can  edit, change status or type, read or reply

to   a  messages......   Simple  move  the  highlited  bar  on  the

intersting messages and press one of thos combination of keys:

ALT-I  The   more  complex,   is  used  to  change  status,  title,

destination  etc... To  move around  the fields, use the TAB

or  RETURN keys.  To end  the operation,  move to  the  <OK>

field and press RETURN. ESCAPE will abort the operation.

ALT-H  If  the message  is in  H state, will became N, otherwise it

will be change in H state.

ALT-K  Kill the message. No other operation is available.

TSTHOST v1.42 (c) IK1GKJ                                     43

ALT-F  All  the messages  in N  state that  do not  have a routing,

will  be add  to forward  list queue. For any other case, if

the  status of  the message is N it will became F, otherwise

change to F state.

RETURN         Read the message

CTRL-RETURN    Read the message, but with the RLINE expanded.

NOTE:  while reading,  you can  move  into  the  message  with  the

classic  direction keys.  Press ESCAPE  to end  the reading session

and return into the list window.

ALT-E  Edit the message.

ALT-R  SEND  REPLY, make  a return  message to  the sender  of  the

selected message.

CTRL-R Like  ALT-R, but  will put the contents of this message into

the text of the reply message.

ALT-C  SEND COPY, make a copy of the message.

ALT-M  Mailcopy, to copy/append the the highlited mail to a file.

NOTE:  ALT-R, CTRL-R,  ALT-C, ALT-M  open  another  window  in  the

screen.  In this  window you  may specify  a filename to add at the

end  of the  new message.  If you do not need to add external file,

simple  press enter on the empty field. Instead, if you need to add

file,  type here  the name. If the filename do not have a specified

path,  this file  will be searched in the WORKDIR directory. If you

do  not remember  were is  the file, you may press the F2 key, that

open another window for a selection from a list.

The  ALT-C command  also have to specify the address and optionally

the  destination bbs  for the message. Use tha TAB or RETURN key to

move in the field.

ESCAPE always abort the operation.

While  editing the  message, the  internal editor  do any  type  of

movement  with the  direction keys  and ist  combination  with  the

CONTROL  KEY. CTRL-Y delete a line. Press F2 to end the editing, or

F8  to abort.  At the end of the edit session, you will enter again

in the list window.

TSTHOST v1.42 (c) IK1GKJ                                     44

* K

For  more simplex  operation, killing  of the  message may  be done

also  directli from  the keyboard.  K command  accept all estension

provided for the L command.

* S  CALL [@ ROUTE] [+FILENAME]

* SB CALL [@ ROUTE] [+FILENAME]

* SP CALL [@ ROUTE] [+FILENAME]

* SR[R] MESSAGE_NUMBER [+FILENAME]

* SC    MESSAGE_NUMBER CALL [@ROUTE] [+FILENAME]

The 'SP' sets the message to Personal,  that  means  that only the

addressee can read the  message.  The  'SB'  sets  the message  to

Bulletin,  that means  that anybody  can read  the message.  If the

'SB'  or 'SP'  are not  expressly specified, (the 'S' command), the

program  assigns the 'P' mode for any valid CALL, otherwise the 'B'

mode.

SR  automatically replies  with a  P-type message  to the sender of

the  specified   message number.  The title  of the message will be

"Re:" and the title of the original message.

SRR  work like  SR, but insert in the editor a copy of the contents

of message_number

SC send a copy of the specified message number to CALL.

During   those   command,   you   cannot   switch the task, but you

must  terminate the  message editing  session. If +FILENAME exists,

the  content   of the specified file is appended to the end of your

message.  If   FILENAME has a drive/path specification, the file is

read  from the  selected location, otherwise it will be searched in

the WORKDIR path.

TSTHOST v1.42 (c) IK1GKJ                                     45

PMS COMMAND FROM KEYBOARD

-------------------------

* ACK [H | Y | N]

This  command enable  the ack  of any message received that contain

the  string /ACK  in the  first column  of oe  of the  line of  the

received  messages. The  ack is  like a  return answer  confim that

your  pms have  received  the  message.  The  confirmation  of  the

reception  of the  message is  automatically returned to the sender

of the message.

Y  enable the  ack function, N disable, H make the ack message, but

put  it in  held state,  this means that the ack message is locked,

and will not be returned until you unlock it.

* BReak [callsign | ALL] text

Send  text to  the callsign  connected on PMS. Using ALL instead of

callsign  will send text on all PMS connected users. The break will

be send only for the pms station that are in idle state.

* ENdmessage filename | OFF

This  command defines  a file  that will  be automatically added at

the  end   of any  message written  with the S?/SR functions of the

PMS.  If the   S?/SR  command already have the +FILENAME extension,

first  will be  added the  FILENAME contents,  then the  ENDMESSAGE

contents.

OFF Disables this function.

If ENDMESSAGE does not have a specified path, it will  be searched

in the program directory.

* FBbmode ON OFF

Enable  (default) or  disable the fbb compressed forward mode. This

type   of  forward  require  about  32  kbyte  of  memory  for  the

compression  routine, if  the program do not found enough memory to

start  the compressed  forward, automatically switch to the MBL/RLI

forward type.

* FREverse ON OFF

Enable  (default) or disable the reverse forwarding. If freverse is

OFF,  tsthost will  call homebbs  ONLY if there are mail to send in

the   pms.  With   freverse  ON,  tsthost  will  call  homebbs  for

forwarding even if there are not mail to send.

TSTHOST v1.42 (c) IK1GKJ                                     46

* FTIME 0..60 minutes

Every  FTIME minutes,  the PMS  will connect  the master BBS asking

for  the   forward. Any value  below  5 disables the function,  and

the forward is performed only if the HomeBBS calls your station.

* FWD

Manual start of the forward with the HomeBBS.

* HAlias CALL-SSID

To  define an extra callsign that will be considered like a homebbs

when connected.

* HIeraddr TEXT | OFF

Is  a field  of max 23 characters, used to define your hierarchical

address, for example .IPIE.ITA.EU

OFF  disable the  function. This  field, if  present, is  used with

RLine command.

* HOmebbs CALL-SSID

Defines  the BBS  where the  PMS messages  have to  be transmitted,

those  with the  routing field  (@) set. SSID must be specified, as

IK1MSL is different from IK1MSL-8. For example:

HOMEBBS IK1MSL-8

NOTE:  if you  change HomeBbs  callsign, and  the unproto  list  is

active,  the list for the previsuly homebbs will be closed and will

be open a list for the new homebbs.

* LBHeld ON | OFF

Any  bulletins locally  generated will  be put in status HELD. This

meand  that only the sysop and the sender of the message may accest

to this mail.

* LPHeld ON | OFF

Like LBHELD, but works on personal mail

TSTHOST v1.42 (c) IK1GKJ                                     47

* MAIlcopy [number... <IK1GKJ >IK1GKJ @IK1GKJ] [file name]

Append   to  filename   any  valid  combination  of  the  specified

arguments.  You may  specify a list of number, a sender callsign if

preceeded  by <,  a destination  callsign (>), a routing (@) or any

comabination  of those parameters. The latest arguments must always

be  the fil  were append the messages. If file name does not have a

drive/path  specification, the  file will be written in the workdir

directory.

* NBID bidnumer (1...65535)

To  assign a new bid start sequence. DO NOT USE THIS COMMAND IF YOU

DO NOT KNOW WHAT YOU ARE DOING!!. Read TSTHOST.BID section before.

If   you  do  this  command  without  parameters,  simple  will  be

displayed the next internal bid number.

* PMS

The current task is switched in PMS mode, for instance after a QSO

with the remote user that called you. The user returns to  the PMS

prompt. You cannot switch if you have any active file  transfer in

the channel (YAPP, record, send). See the TALK command.

* PYAPP [ON | OFF]

Enable disable yapp function of pms.

* RIng [ON | OFF]

Ring  work with  the PMS  command T,  used by  an user  to call the

sysop. If ON (default), calling sysop is enabled, if OFF no.

* RLine ON | OFF

If  ON, when your PMS forward a message, your R: line will be added

to  the message,  if OFF  no. WARNING:  with RLine  ON, you will be

considered  exactly like a regular bbs, so, if a user use a command

SR  on your  mail, the  return address  will  be  YOURCALL@YOURCALL

insted  of YOURCALL@YOURHOMEBBS. If @YOURCALL is not defined in the

homebbs  and/or adiacent  bbs inhis  forward file, mail sent to you

vill  nevar arrive  to you,  this because  you are not a bbs in the

regular  path. Whit  off this  is not  a problem, because your mail

will be returned to your homebbs, and homebbs is know in the path.

But,  some server have the capability to communicate one to another

trougth  the RLine,  and in  this case,  if you do not enable RLine

this type of server cannot work properly. So see you need.....

TSTHOST v1.42 (c) IK1GKJ                                     48

* SAvemsg +/-TOFIELD +/-@ROUTE +/-<FROMCALL .... up to 20

To  define up  to 20  address field. Whe a message will be received

on  the pms  interface, if its TO , @ or FROM field match one field

in  the list, the message will be copied into a file in the WORKDIR

path,  with .MSG  extension and TO name if match to, @ROUTE name if

match a route, #FROM name if match a from field.

* SERver +/-SERVERNAME +/-SERVERNAME .... up to 20

To  define up  to 20  server program  that will  be  executed  when

arrive a personal message addressed to SERVERNAME@YOURPMS.

+  ADD server,  - REMOVE server from list. Do not specify extension

or  path to  the server  statement,  only  the  filename,  and  the

executable file must be located into the TSTHOST.EXE directory.

* TALK

The  current task is switched  to  the  normal  conversation mode,

for  example when  the remote  user TALKs the sysop (see PMS). You

cannot switch if the PMS is not in stand-by status.

* Timeout minutes

Specify  the time after, without activity on  the channel,  i will

be automatically disconnected. Time is between 0 and 60 minutes; 0

(default) disables the timeout function.

* UPdatedms pday bday pfday

When  a message  is killed, the file is not  deleted from the disk

but the status flag becomes 'K'. This command allows  the deletion

from  the system  of those  messages that are killed, and to define

how  time a  message may  reside on your pms become of an automatic

deletion.  The parameters  are the  number of  day from  the  today

date, to the date of the creation of the messages.

The  first parameters  is referred  to a  personal message not in F

state.  The second  are for bulletins, any state. Next the personal

mail in F state.

Messages in K state are always immediately deleted.

Messages in H state is nevar deleted.

Please  note that  you cannot run  this  command  if  any  user is

connected to the PMS interface.

TSTHOST v1.42 (c) IK1GKJ                                     49

For   example,  to   erase   all   private messages  after 10 days,

bulletin messages after 4 days, and PF after 3 day:

UP 10 4 3

* USerdir [path] [path] [path] [path] [path]

Sets the directory for the users connected to the PMS interface.

The default path is a subdirectory within  the  one  that contains

tsthost.exe, but you can change to a new path with this command.

Note that TSTHOST does not CREATE  the  directory,  the  path MUST

EXIST.

Is  possible to  define up to 5 path, that a user connected to your

pms,  may access  via the virtual drive specification letters A: B:

C:  D: E:.  The first  path is  identified like drive C:, ma is the

defualt,  and drive letter may be omitted. For example, if the user

do  the command  W E:,  this users  will access  to the 3 specified

path.

NOTE:   An  user,  after  a  succesfully  sys  command,  that  have

permission  to read  or write  on all  your system,  see the really

drive, not this virtual drives.

TSTHOST v1.42 (c) IK1GKJ                                     50

PMS commands for remote users

-----------------------------

NOTE:  ANY COMMAND, BEFORE TO BE DECLARED UNKNOWN, WELL BE SEARCHED

LIKE  A PG,  THIS PERMIT TO EXPAND THE PMS COMMAND WITH PG PROGRAMS

WRITTEN FOR PARTICULAR FUNCTION.

* R [XR]  to read messages

* K       to kill messages

* L       to list messages

The valid parameters for the R, L, K commands are listed in the

following table (the examples are for the List command) :

L              List all messages (only LIST, not READ/KILL)

Will be displayed only the message received after

your last list command.

LL 50          List last 50 message (only LIST, not READ/KILL)

L 100          List message number 100

L 100 102 150  List messages 100 102 and 150 (max 10 number)

L 100-150      List message from 100 to 150

L -150         List message up to number 150

L 150-         List message from number 150 on

L< CALL        List all messages sent by CALL

L> CALL        List all messages for CALL

L@ ROUTE       List all messages for that route.

LS STRING      List all messages that have STRING in the title.

LM             List all my messages

NOTE:   Any  users have  access  to  all  bulletin    and  personal

messages  addressed to his callsign or that  he  have  sent. Killed

messages will never be listed.

XR  work exactly  like R, but while R compress the R line like fbb,

XR display those line completely.

* B

Disconnects the system.

* DOS DOS_COMMAND

If enabled, TSTHost executes the  specified DOS command.

TSTHOST v1.42 (c) IK1GKJ                                     51

* I

List the users connected to the PMS interface.

NOTE:  Only the stations connected to the PMS are listed. Channels

not  in PMS-mode are not listed. Display also the available virtual

drive.

* N NAME

Register your name.

* PG [[NAMESERVER] SERVER PARAMETERS]

If called without  arguments, the  program sends  to the  user the

contents   of  the   file TSTHOST.PGS, if exist, otherwise sends NO

PG  AVAILABLE. In  this way  it is  possible for  the sysop to have

some  PG servers  not declared. A user will not execute the servers

but  the sysop,   or  a  restricted  group  of  users,  can  easily

activate these extra services.

If  the server  name parameter  is specified,  the program executes

the  specified  server. The  output of  the server  is sent back to

the user.

* SYS

When   the  user  performs  the SYS command,  his callsign  will be

searched  in   the TSTHOST.SYS  file, and,  if  found,  the  system

requires  a password,  codified with  the standard five letters and

also  with the  MD2 system. The user may answer with one of the two

modes.  If the user enters the corrected password, to the user will

be  assigned some  privileges, specified in tsthost.sys file by the

flag parameters. See tsthost.sys section for info about flags

The  password prompt is the PMS callsign without SSID,  with _PMS>

and the password request, like:

IK1GKJ_PMS> 5 1 3 7 6 [0123456789]

To  cancel previusly  authorization, you may repeat the sys command

and  do a  bad  or  null  password.  If  you  change  the  flag  in

tsthost.sys  while a  user with a valid authorization is connected,

the  new flag  will not have effect until the current authorization

will not be cleared.

* S  CALL [@ROUTE]   * SB CALL [@ROUTE]           * SR MSG_NBR

* SP CALL [@ROUTE]   * SC MSG_NBR CALL [@ROUTE]

Stores  a message  into the   PMS.  SB is for bulletin messages, SP

for  personal  mail. The  field @ROUTE  is optional.  If specified,

the  message will  be forwarded to the specified HomeBBS. If  the P

TSTHOST v1.42 (c) IK1GKJ                                     52

or  B types are not expressly specified,  the program automatically

assignes the P type for  any  valid  CALL, otherwise the B type.

SR replies a P-type message to the sender of a specific message.

SC send a copy of message number # to call.

To terminate the message editing, send CTRL-Z  or /EX in the first

text column.

* T

Calls  the sysop  of the connected PMS. You are always connected to

the PMS interface until the sysop do not switch it in TALK mode.

* V FILENAME

Display  to the  users the  contents of an ascii file. File must be

in USERSDIR or its subdirectory.

* YD FILENAME

Sends  to the   user  the specified  file using  YAPP protocol. The

file must  be in  the  USERS  (USERDIR  command) directory or  its

subdirectories.

* YU FILENAME

TSTHost receives a file from the user, with the YAPP protocol.

The Resume option is always enabled, so if the transfer  stops for

any reason, the user can reconnect the system and restart the file

transfer from the  point where  he was  arrived. The file will  be

put in the USERS\UPLOAD directory.

* W [PATH]

Display the contents of the USERDIR directory.

* ?

Sends to the  user the  contents of  the TSTHOST.HLP file. If this

file   do  not  exist,  the  user  receives  the  message  NO  HELP

AVAILABLE.

* @ [CALL | ALL] TEXT

Interrupts  the  user  CALL,  that  must  be  connected to the PMS

interface,  by notifying  the TEXT  message. If  you use  ALL  like

callsign, the break will be sent to all pms channel.

TSTHOST v1.42 (c) IK1GKJ                                     53

UNPROTO LIST

------------

When a FBB BBS receives a new  message,  emits  a  special Unproto

frame  with the  message description.  Also, it  is possible to get

the BBS message list without connecting the system.

With  those  features,  a  user with the PC active all the day can

receive and maintain the list of the messages present on the BBS.

It is not needed to connect the BBS to get the message list, too.

To  use this feature  you must be enabled on the BBS system so, if

you need the Unproto list, contact the Sysop.

Even  if you are not enabled, the program  can  create  a  list by

intercepting  the Unproto  messages, but  cannot resyncronize  with

the BBS system if a packet is lost.

NOTE: to use the unproto list, monitor channel  must  be  open for

the  Unproto frames,  letter U.  AX MON  U,  AX MON US, AX MON UC,

AX MON USC are correct, some other modes without the 'U' aren't !

The  Unproto list can save in memory up to 800 message headers, but

to  avoid memory  consumation, you may specifi the unproto size via

the /U command line parameters.

The  HOmebbs  command  sets the BBS from which the Unproto list is

accepted.  You must  specify  the callsign SSID, because IK1GKJ is

different from IK1GKJ-8.

You can set up  to 20 fields that are automatically  selected when

received,  like is possible to set up to 20 fields that will not be

inserted  in the  list. The  list  can  be  obtained  without  user

operations,  or by  issuing   a special  command (please  read  the

section  about channel  8 on  the manual), even if the BBS requires

an   access  password.   Unproto  list   is  transferred  with  the

compressed  FBB protocol.  This protocol  requires about 19K of RAM

for the message decoding.

The  received files  are then  put into  the PMS  interface, and me

next  view with  the L  command, that access to the pms function of

tsthost.  The  received  file  have  the  same  proprierty  of  the

standard  file received  in forward,  so may  have  /ACK,  SAVEMAIL

function etc...

TSTHOST v1.42 (c) IK1GKJ                                     54

Unproto  List is  directly connected  with  the  HomeBbs  callsign,

since  is this  callsign  that  generate  the  unproto  frame.  The

information  received by  tsthost, is  stored in  a file  that have

".ULS"  like extension,  and the  call without ssid of your homebbs

like  name. File  is stored  in the  tsthost.exe directory.  So  is

simple  have multiple unproto list. If you receive for example, two

list,  from two  bbs, when  you change  homebbs tsthost  change the

list information, and are ready to work with the other bbs.

IMPORTANT NOTE

Some  programs allow the use of unproto frames with  the  poll bit

set, that is not a  problem  for  the  Unproto  list.  However, in

certain conditions, this can become a problem.

For  instance, you  are connected  to your  HomeBBS on  channel  1.

TSTHost  emits   a Sync   Unproto  frame  towards your HomeBBS, for

the  Unproto  list.  The  HomeBBS, when receives the Unproto  frame

with  the poll  bit set, returns you  a  DM  frame. This is correct

since your monitor channel is not connected.

But  your AX25 interface receives the DM frame ,  and  then  it is

redirected to the channel 1 where you are connected, to disconnect

the   link.  In this  case you  are disconnected by your TNC, while

for the BBS you are still connected.

To  avoid this  problem, TSTHost  starts by sending the @U0 command

to  disable the  poll bit  in unproto  frames. Maybe some Eproms or

Driver  do not recognise the command (as far as I know, only  TFPCR

v2.10)  and if  the Eprom/Driver  starts with  the poll bit set you

cannot  fix the  problem so  you'll be randomly disconnected by the

system.

TSTHOST v1.42 (c) IK1GKJ                                     55

UList Subcommands

-----------------

Use these commands after the ULIST prefix, for instance:

ULIST QUERY 122

* Autoselect +/-TO_Field +/-<FROM_field +/-@ROUTE_field

Automatic selection of the messages in the  Unproto list according

to  the desired  TO_Fields (up  to 20). For instance, to select the

ALL, DX and NEWS fields the command is:

UL A +ALL +DX +NEWS

Then, to add the KWD field and remove the ALL field:

UL A +KWD -ALL

With  the same  format of  above, you may select the FROM field; to

do  this, you must add the prefix '<' to the sender of the message.

For example, UL A +ALL +<IK1GKJ

UL A  without parameters displays the autoselect list.

* Disable

Disables the  Unproto list. The  Unproto frames are not decoded to

the list. The list is saved in the TSTHOST.ULS file in the TSTHost

default  directory. The memory used for the list is released. NOTE:

the list will be automatically saved every hour.

* Enable

Activates the Unproto list.  The previous list is loaded from disk

(TSTHOST.ULS), then any new message is added to the list.

Being  enabled by  the HomeBBS, a missing message activates a synch

request so to keep the list always updated.

* Get

Connects the HomeBBS to receive the selected messages.

* List

To display the unproto messages selected and not yet received.

TSTHOST v1.42 (c) IK1GKJ                                     56

* Merge filename

The  purpose of the command is to merge the contents of the unproto

list  in memory  with an unproto list on a file. In this manner, is

possible  make  server,  program,  or  other,  that  transform  the

standard bbs connection list, to an unproto list compatible.

The  union work in conjunction with all the filtering selection for

the  active unproto list, autoselect, notselect, pmsg..... Filename

will  be searched  for default  in the  workdir, if  a path  is not

directly specified.

Order,  or not  order, for  the message   in the merge file are not

important,  they will be sorted with the unproto in memory. But the

file  must have  a unproto  format compatible,  data must be stored

exatcly like if they was received via radio.

Here  is the  format for  the unproto  list, and the format for our

file.  Lines cannot have more of 80 characters, any line invalid is

discarded, any valid line have the form:

NUMBER TYPE SIZE TO [@ROUTE] FROM DATE TITLE

|      |    |    |  |        |    |    |--title of the message

|      |    |    |  |        |    |--date of the msg, es:DDMMYY

|      |    |    |  |        |--call of the sender without ssid

|      |    |    |  |--routing, optional, no ssid

|      |    |    |--addressee of the message, without ssid

|      |    |-- a number, is the size of the message.

|      |-- type, a characters, may be P,A,B,T

|-- number of the message.

Right example:

123    P    3214 IK1GKJ@IK1MSL IK1VCQ 110293 TEST

124    B    1233 TUTTI @ITA    IK1MSL 110293 TEST 1

125    P   12345 IK1MSL        IK1GKJ 110293 TEST 2

You must do the right number of element and correct spaces

* Not +/-TO_Field +-<FROM_field +/-@ROUTE_field

Work  exactly like  Autoselect, but  the fields  set into this list

will not be inserted in your unproto list.

* Pmsg ON | OFF

If  OFF, only  personal mail  addresses to  your callsign  will  be

added  to the  unproto list,  other P mail will be discarded. If ON

all P mail will be added to the unproto list.

TSTHOST v1.42 (c) IK1GKJ                                     57

* Query [msg_nbr]

Manual  request to  the BBS  to send  a  list  from  the  specified

message  number. This  command should  not  be  needed,  since  the

program  is always  automatically synchronised.  If msg_nbr  is not

specified,  automatically the  program ask  for the  last  received

message  in the list. NOTE: if a manual or automatic QUERY will not

receive an answer, after 1,5 minuts the program will ask again.

* Rpath [OFF | digipeater list]

Normally  you do  not need  to use  this command,  may be some case

where  the connection  bbs have  a digi  list, and  resync  request

another  path. If  you define  a digipeater here, the connection to

homebbs  will be  executed with  the path defined in HBBS and HNODE

command,  but the  RESYNC request  will  be  addressed  to  homebbs

trought this path.

* Time 0..60 minutes

This  command    sets  the  time  interval,  in  minutes,  for  the

automatic  connection. If  there is   any message selected, TSTHost

connects  the HomeBBS  for the  message request.  Any value below 5

min.  disables this feature.

* View [<CALL] [>TOCALL] [@ROUTE] [title substring]

Displays  in a  window the  message list.  Use the  standard cursor

keys  to scroll  the list,  INSert to  select/deselect  a  message,

ESCape  or RETURN to close the window. A "-" displayed in the first

column,  means that  the message was already present at the time of

the  last view command, an "R" means that the message was requested

and   received  from  HomeBbs.  "X"  means  that  the  message  was

requestes to HomeBbs, but was not available.

Whitout  parameters, all  the  list  will  be  displayed.  You  may

specify  to list  message only  FROM a  call (prefixed  by <), TO a

call  (prefix >)  or a  route (prefix @). If you pass any arguments

that  is not prefixed by ><@, the program display only headers that

have that arguments in the title.

TSTHOST v1.42 (c) IK1GKJ                                     58

TstHost commands description

----------------------------

The uppercase letters are the minimum command abbreviation for the

command recognition. Facultative parameters are  shown  in  square

brackets, while mutually exclusive options are separated by |.

Send,Delete,Dir,Yput,View parameter window

------------------------------------------

If the parameters for the SEND, DELETE and DIR commands  have  not

been  specified,  the  program  activates  a request window on the

screen showing the files contained in the WORKDIR directory.

Select  the  desired  file  with  the  ENTER  key,  by  moving  the

selection   bar  with   the  cursor  keys. To log to another drive,

press  the TAB key and type the corresponding  letter.  The  ESCape

key  aborts the  request. The  ENTER key  has no  effect in the DIR

window.

Option that include a directory display

---------------------------------------

All  the option  that display  a directory  contents on the screen,

have  the capability  to sort  the directory  contents  in  several

mode.  Simple with  the directory displayed press CTRL-N to list on

filename+extension  order, CTRL-E  for  extension+filename,  CTRL-D

for  date, CTRL-S  for file  size,  CTRL-U  unsorted.  The  program

remember  the latest  option selected,  and use  it again for a new

directory list.

* 7PCopy ON | OFF

If  ON, any message arrived in your pms, or with unproto list, that

have  any valid 7plus code, will be copied into the file 7PDATA.FWD

in  the WORKDIR  path. This  file is  in the  standard  MBL-RLI-FBB

forward-export  mode, and  may be  processed from server or prigram

that recognize this format, or directly from 7plus etc......

NOTE:   to  avoid  duplicated  reconstruction  of  programs,  7plus

messages  imported via  the tsthost.in file will not be copied into

the 7pdata.fwd file.

* AUtoyapp [on|off]

When  activated, TSTHost  starts automatically  the  YAPP  protocol

when  the init  protocol code   is   received  from  the  connected

station,  otherwise you  have to  start the  file download with the

YREC command. The file is saved in the WORKDIR directory.

TSTHOST v1.42 (c) IK1GKJ                                     59

* AX25 subcommand command

TNC setup command, read the description further on.

* CLS

Clears the receive screen for the active channel.

* CLRbuf

Clears the receive buffer for the active channel.

* COLor process_number color_number

Selects the color for the screen display. Read  the  help  text by

issuing this command without parameters.

* Connect callsign [v digipeater digipeater]

Connect the specified callsign on the current channel.

* COPy source destination

Copies the source file in the  destination name. If a path name is

not specified, the default directory is the WORKDIR. Wildcards are

not allowed, and you cannot  overwrite an existing file.

* DElete [filename]

Deletes  the file.  If  no drive/path  is  specified,  the default

directory is the WORKDIR path. Wildcards are not  allowed.  If the

filename  is not  specified, TSTHost  provides a  screen window for

the selection.

* DIR

Directory  display  in the screen window. The directory start from

the  WORKDIR directory.  If you press ENTER on a file name, you may

automatically  start an  external dos  program selected by the file

extension. See the documentation about the TSTHOST.EXT file.

* DISconnect

Disconnects the station.

TSTHOST v1.42 (c) IK1GKJ                                     60

* EBell [bit field value]

This  command set  all the acoustic segnalation of the software. Is

a  bit field value: this means that you must add the specific sound

value to obtain the desidered configuration.

1=Bell active on rx data (ex BELL)

2=Bell active on connect/disconnect (ex CBELL)

4=Bell active on end of yapp activity

8=Bell every 10 seconds if exist unread data for our connection

Default  value is  6, bell active for connection and yapp activity:

this because 6=4+2.

* ECho [on|off]

If enable,  the  text entered from the keyboard is also written in

the receive window,  in the receive buffer and (if active)  in the

capture file.

* EDit [filename]

Permits  to edit  or create a text file. If a specified path is not

present  in filename, WORKDIR is assumed. If no filename follow the

command,  TstHost provide  a screen window for the selection of the

file.

* EXIT [parameters]

EXIT      Whit no parameters, stop the program only if there are

not connected channels.                 EXIT CODE 0

EXIT D    Disconnect all channels, empty buffers, and next quit

from the program.                       EXIT CODE 1

EXIT W    Wait for all channels disconnected, disable new users

connection, next exit.                  EXIT CODE 2

After 30 minuts from the activation of the command,

if there are again users connected channels, the

program will force the disconnection.

EXIT X    Immediately stop. No action.            EXIT CODE 3

EXIT R    In case of wait to exit, (EXIT W), you may cancel the

exit request withthe EXIT R command.

NOTE: While wait to exit, the program name in status line blink.

* FUnction function_key(1-20) [command.....]

It   is   possible   to  define ten  commands (up  to 80 characters

long)  that will be executed by depressing the keys  CTRL + F1/F10.

Issue  the   command  with  the  function key number to display the

current  assignment. For  instance,  FU 1 CON IK1MSL-8  assignes to

CTRL-F1   the  connect command to IK1MSL-8. Value from 1 to 10 will

be  associated to  CTRL-F1..F10, value  from 11  to  20  to  SHIFT-

F1..F10

TSTHOST v1.42 (c) IK1GKJ                                     61

* Help or ?

Displays a command list.

* LOg [on | off]

Enables/disables the connection  record  log,  saved  in  the file

TSTHOST.LOG  in the  program directory.  Every  line  in  the  file

contains  the callsign,  time and  date   for any  connection   and

disconnection.   To avoid that this  file  grows  too much,  it  is

suggested   to   move   and   rename  the file in another directory

for further storage every month.

NOTE:   the  file   data  is   valid  only   if   connections   and

disconnections  happen   while   the programs are active (including

the  shell command  RUN). If  the program  starts while  the TNC is

already  connected, the  missing data  are logged  in the  file  as

question marks (??).

*MHeard [on | off | clr]

Similar  to the  AX MHEARD  command, this command is used to enable

(ON)  or disable (OFF) or clear (CLR) the internal mheard list. The

list  have a fixed size of 15 callsign, and naturally may work ONLY

if  the monitor channel is OPEN. If you for example set the monitor

for  only the  U frame, the list is update only when any U frame is

received,  because the  tnc send only this type of frames. Internal

MHeard  list is  supplied only for that tnc that do not have a list

inside.   For  tnc  capable  to  manage  the  list  internally,  is

preferable  the use  of the  AX MHEARD,  maintaining MHEARD  OFF in

order to save cpu time.

* MPerm [ON | OFF]

When   you  activate   this  command  (ON),  a  little  windows  is

maintained  in the  channel 1..8.  In this  window the program will

write  the monitor  packets. Pratically, this is the same data that

normally you see on the channel 9, monitor channel.

OFF disable the function.

NOTE:  Monitor data  type and  activation  are  done  with  the  AX

MONITOR command.

TSTHOST v1.42 (c) IK1GKJ                                     62

* PARam channel text

Use   this   command  with care,  only if  you know  very well  the

WA8DED  host mode software. This command set a specific value for a

channel  or   an    extended  command  not  included  in  the  AX25

subcommands  using the   host  mode commands,  and the  data is not

checked  by TSTHost.  It is  used for  particular settings  or non-

standard  Eproms. The  user is responsible  for  any  data  sent to

the  host interface.  The global communication  channel  is  number

0,   the   others   from   1   to  8 correspond  to  the eight task

windows.  Notice that  data written  on channel  0 is stored as the

default,   while   those   written  on  the other channels are used

until  the next  disconnection on  the channel  and return  to  the

channel 0 default value.

* PASsword [CALLSIGN]

Automatic transmission  of  the  requested  password.  Support the

standard  request and  the MD2  algorithm. It  is also  possible to

send  the password  with the  ALT_F1 key, that is the equivalent of

PASSWORD command.

If  CALLSIGN is  not specified,  the password  will be searched for

the  callsign  known  in  the  actual  tnc  link,  if  CALLSIGN  is

specified,  the password  will be  searched for  the provided  call

insted of the tnc link call.

* RECord filename | off

Captures to a disk file the task activity. The file  logging stops

with  the OFF  filename or with the task  disconnection. The status

line  displays the  disk file  size  and  not  the  received  bytes

number  because   logging to  an existing  file appends data to the

end of the file without deleting the previous text.

* REName old_name new_name

Change  the name for the specified file. Wildcards are not allowed.

The   default  directory is  WORKDIR. It is possible to move a file

in  the same  disk to  a different  directory by specifying the new

path. For instance:

REN  pippo.txt pluto.txt

The file pippo.txt becomes pluto.txt

The file remains in the default directory.

REN  e:\bin\utils\vp.exe e:\bin\vp.exe

moves the file vp.exe from the path e:\bin\utils into

e:\bin

TSTHOST v1.42 (c) IK1GKJ                                     63

* RUn [filename]

Executes  the  Filename  DOS  program. If  the program name is not

specified, the command interpreter (COMMAND.COM) is run  until the

EXIT   command,   that   returns  to  TSTHost. The TSTHost activity

is suspended while executing DOS programs.

*SCReensave [Minuts 1..60 | TEST | OFF]

OFF disable screen saver

TEST activate it immediately

Any  numeric value  tell tsthost to activate the screen saver after

the specified time with no activity on the keyboard.

* SEnd [filename | OFF]

Sends the specified file to the connected station. OFF  closes the

file, or  it  is closed at the end of the file or when the task is

disconnected.  The file  must contain  ASCII text  and  not  binary

data,  and the  SEnd command cannot be issued in a PMS session. The

status  line   counter   displays the  number of  bytes sent to the

TNC,   so the file  can  be  closed when the TNC still have data in

the  buffer  to  be  transmitted.  If  the    filename    is    not

specified,  a  selection window is displayed on the screen.

* SOurce filename

Executes  a command  file. The  file contains  the commands  in the

same  form   as   for   the    keyboard.    Each  line  (up  to  80

characters)  must contain  a command.  The source  file is searched

ONLY in the  program directory.

* TNCDelay X Y

This   command   set  some program timings, that you shouldn't need

to  change. The  values are  expressed in  18th of  second and  the

range is from 3 to 255, that correspond to 2 tenth and 14 seconds.

The  X parameter  is   the maximum  delay for the TNC reply. If the

TNC  does   not   reply to  TSTHost   in  the  specific  time,  the

program  retries to synchronise the serial port. The Y parameter is

the speed for the serial port synchronisation.

If   your  Y parameter  is too  high, you  can go  to have a coffee

while  the program  resynchronises! You  must  increase  X  to  fix

frequent  timeouts, and  Y if  the program cannot resync the serial

port.

IF  YOU ISSUE THE COMMAND WITHOUT PARAMETERS, WILL BE DISPLAYED THE

CURRENT TIMING VALUE AND A STATISTIC OF THE PROGRAM ERROR.

TSTHOST v1.42 (c) IK1GKJ                                     64

* UList subcommand command

Command  for FBB  Unproto list,  read   the   specific  section  in

this manual.

* View [filename]

Display  the contents of an ascii file, predefined path is WORKDIR.

If  you do  not provide  a filename,  a selection is made in window

mode.

* WOrkdir [path]

Set the default work path for TSTHost.  Usually, it corresponds to

the  directory where  the program  is stored.  When a filename does

not  have a  drive/directory path,  it will  be   searched  in  the

WOrkdir directory.

* WRitebuf filename

Writes  the contents  of the  receive buffer in an ASCII disk file.

If  the filename does not have a drive/directory path specified, it

is saved in the WOrkdir directory.

* YABort

Starts the procedure to interrupt the YAPP transfer protocol.

The  interruption is  not immediate  because the  terminal  station

must  empty   his   buffer   before   accepting   the   command. To

perform  an immediate abort, type this command TWICE.

* YAPpc [on|off]

Enables/disables  the YAPPC  protocol version.  You  cannot  change

this option if there is any YAPP file transfer active.

* YRec filename

YAPP  receive protocol  manual  start.  If  Autoyapp  is  on,  this

command  is not  requested. If  Autoyapp is  off, you  must execute

this  command just  after having  received by the connected station

the   Init  YAPP transmit  code. If  the filename  does not  have a

drive/directory  path specified,  it will  be saved  in the WOrkdir

directory.  This command is not active in a PMS session.

* YPut [filename]

Start  the   YAPP transmit  protocol. If  a drive/directory path is

not  specified, the  file is  taken  from  the  WOrkdir  directory.

TSTHOST v1.42 (c) IK1GKJ                                     65

This  command is   not  active   in a  PMS session. If the filename

parameter is not specified, the program activates a select window.

* YTimeout [seconds]

To   specify  the  time  to  wait  without  receiving  data  before

declaring timeout in a yapp session. Default 150.

AX25 Subcommands

----------------

These   parameters   must   follow  the AX25  command, for instance

AX25  MONITOR N.  The TNC manual reports  more  informations  about

these commands.

* AX25l2v2 [On|Off]

Enables the level 2 of the Ax25 protocol.

* Beacon [0-60] [Beacon Text | <Beacon file]

Set   the   beacon   text.  The time value is expressed in minutes,

and  any value  below 5  disables   the   beacon.    If  the  first

character  of the   beacon   text   is '<', then the others are the

text  filename. If  no drive/directory  is specified, then the file

will be  searched in the TSTHOST.EXE directory.

Example for text beacon:   AX BE 30 ST. IK1GKJ * IJ45IK

Example for file beacon:   AX BE 30 <F:\UTILS\BEACON.TXT

* Check [0..65535]

After  this time,  if was  not detect  traffic on  the channel, the

program test the integrity of the link. In 10ms.

TSTHOST v1.42 (c) IK1GKJ                                     66

* CLock ON | OFF | SET

The SET parameter sets the current time/date on the TNC.

ON enables the time/date display in the monitor window and  in the

status  messages, OFF  disables this  time/date display.  NOTE: Not

all  TNC EPROMs  accept this  command; it  works fine  on TF24  and

TF26.

* CText [Text | OFF]

To  define a  text that  will be  automatically sent to the user at

the connection time.

* Digipeat [On|Off]

Enables/disables the digipeating capability of the TNC.

* FRack [1..65535]

Pause  (if ACK  has not  been received)  for the TNC before sending

the  data packet  again. This  value can  be from  1 to 15 seconds.

Some  TNC EPROMs  accept valid  data from  16 to  65535,  expressed

in 1/33  of second.

* FUllduplex [On|Off]

Enables/disables the full duplex transmission of the TNC.

* MAxframe [1..7]

Max number of data packets to transmit before waiting for ACK.

* MHeard [ON | OFF | CLR 3..MAX]

Without parameters, displays the list of the received stations.

CLR clears the list.

ON  enables the list automatic update.

OFF disables the list automatic update.

The  list size,  set by  CLR, must be a value between 3 and the max

supported  by the  tnc, but  warning, since  the mheard list is not

maintained  in memory, but only displayed, you you ask for a number

greater  than the  screen dimension, due to the scroll you may lose

the  first callsign  displayed.  Warning:  This  command  has  been

tested  with EPROMs  TF24 and  TF26, but  not any EPROM accept this

command.

TSTHOST v1.42 (c) IK1GKJ                                     67

* MYcall callsign

Set my callsign.

* MOnitor [NIUSC] [+- callsign callsign...] [/F]

N disable

I display I  Frames

U display UI frames

S display Supervisory frames

C display Connections frames

+ Include max. 8 callsign

- Exclude max. 8 callsign

If  the /F  switch is set, the program will continue to process ani

type  of data  received, but display only the header information of

a  pcket, without the data. To restore the original monitor display

option,  you may  do again  the command  AX MON  /F, or  set a  new

monitor mode.

* PAclen [80..250]

Byte size of the transmitted packet.

* PErsist [0..255]

Transmit  chance. When   the  TNC  must transmit data, it generates

a  random   number between 0 and 255.  If the number is higher than

the  PErsist value it can transmit, otherwise  the  TNC  must  wait

for SLottime time before generating a new number.

* POrtcall [channel 1..8] [call|OFF]

This  command  assigns  a  different  SSID  or  callsign  for every

channel.   The  command without  parameters displays  the  list  of

those  channels with  a callsign  different from  the  AX25  mycall

value.  OFF   sets  the  channel  to  use  the callsign-ssid set by

the AX25 mycall command.

To   activate   the  default callsign, use the OFF parameter. Note:

a  channel with  a particular  callsign will  not  change  even  by

changing  the global callsign. To disable the extended callsign use

OFF.  This command  has effect,  for a  connected  channel,    only

after  the disconnection

Example: AX25 MYCALL IK1GKJ

AX25 PORTCALL 2 IK1GKJ-2

TSTHOST v1.42 (c) IK1GKJ                                     68

* PTt [ON | OFF]

Enables/disables the TNC PTT line. The  software  works correctly,

but, while in the OFF state, the TNC  does  not  activate  the PTT

line.

* RESptime [0..65535]

Wait time before ACKing an incoming packet, in 10ms.

* RETry [0..127]

Retry  number before  failure and  station disconnection.  0 stands

for forever.

* Slottime [0..127]

See PErsist, in 10ms.

* Txdelay [0..127]

Delay   before   data   transmission after the PTT line activation,

in 10ms.

* UNproto [text]

Unproto text and route where the Unproto messages are addressed.

* USers [0..8]

Max number of incoming connections.

* Validcall [On|Off]

Enables/disables the valid callsign control.

TSTHOST v1.42 (c) IK1GKJ                                     69

THE STATUS LINE

---------------

From the bottom to the top of the screen:

The  last screen  line displays  the channel  status, the number of

bytes  transmitted for  send file,  the disk file size for incoming

text  capture. Only  the TNC  status and the number of free buffers

are displayed for Monitor and Unconnected channels.

* FR is the number of free buffers in the TNC (or TFPCX/R driver).

Every buffer contains 32 bytes. This values helps detecting if the

TNC   is   collapsing,   as  the Host software disables the monitor

if  this value  is under  256. The  TNC collapses  when the  FR  is

around  128. Among   the   various   reasons,  check the high speed

traffic  and slow    serial   connection  baud rate,   the  monitor

active   with  multiconnection, etc.  The free  buffers   value  is

global and not referred to a single channel.

*  BF is  the number  of packet  in the TNC queue to be transmitted

for  the channel.  TSTHost has a ten buffers queue. If the FR value

drops  below 300,  the queue  is reduced to 5, and if FR goes below

210 then the queue is reduced to 2.

This  function does  not  interfere with the MAXFRAME 7 parameter,

since the correct number of packets is BUF+UNACK.

* UN is the number of packets transmitted still to be confirmed.

This value depends on AX25 maxframe.

* RE is the time of the TNC attempts to  transmit  the  unack data

without   receiving   ACK   from   the   other  station. After AX25

retry value, the station will be disconnected.

* US is the amount of memory used by the system

* OK is the free memory in the system

* TX is displayed only by sending a file. This value is the number

of bytes read from the file and  transferred intothe TNC. When the

file transfer terminates on the PC side,  it  can  still  continue

because of the TNC buffer.

* RX is displayed only when the RECORD command is active, and this

value corresponds to the disk file size.

Next line up on the screen  displays the TSTHost status:  CONV for

conversation  mode   (typed data  is sent   to  the other station),

CMD   for  command   mode  (enter   commands  for   TSTHost  to  be

interpreted).  On the  right side  there  is  the  current  channel

number and the callsign and duration of the active connection.

TSTHOST v1.42 (c) IK1GKJ                                     70

Then  there is a number that display the number of resynconization,

and  the   program   version.  REVIEW  indicates  that the revision

buffer  is activated  (PageUp key);   PMS  for the channel set in a

PMS  session (default)  or active in a PMS session, MAIL means that

you  have received  new mail  for you.  A single  M blinking, means

that there is a macro active on that channel.

Above  these lines,  there is  a line  containing the  callsign for

each  connected channel, from 1 to 8. If there is new data that you

have not yet read from a channel, TSTHost will flash the callsign.

Another line is for the YAPP transfer status line, active  only if

the channel is running a YAPP file transfer.

EXIT CODE

---------

TstHost, at the end of execution, return this code:

0, if stop for EXIT command.

1, if stop for EXIT D command.

2, if stop for EXIT W command.

3, if stop for EXIT X command.

4, if stop for any unrecoverable error.

HOW PRINT A FILE

----------------

Any  command related  to a  registration of  a file, like MAILCOPY,

RECORD,  WRITEBUF.... may be redirected to the printer. Simple, use

like  filename the name of a standard output DOS device, such LPT1,

LPT2, PRN..... Example MAILCOPY 22 LPT1

Remember that you print is slow, it may be slow down tsthost.

TSTHOST v1.42 (c) IK1GKJ                                     71

SMARTDRV, DISK CACHE

--------------------

Even  is not indispensable, disk caching programs like smartdrv may

increase  the performance  of the  program. Since the tsthost files

is  not maintained  in memory, but is always read birectly from the

disk,  a cache program may speed up the program activity. If you do

not  have other reasons, the use of smartdrv or other cache program

is highly recomended.

FILE AND MESSAGES VIEWER

------------------------

The  same viewer used to display the contents of a file, is used to

display  the contents  of a  PMS messages.  The viewer  may  handle

unlimited  size file  and messages, but the max line size is set to

255  characters: if a line excedeed this size, the rest of the line

will be displayed on the next row of the screen.

Viewer available commands:

CURSOR UP      One line up

CURSOR DWN     One line down

HOME           Go to top of file

END            Go to bottom of file

CTRL-HOME      Go to top of file

CTRL-END       Go to bottom of file

CTRL-PAGE UP   Go to top of file

CTRL-PAGE DWN  Go to bottom of file

CURSOR LEFT    Move the video one column left

CURSOR RIGHT   Move the video one column right

CTRL-LEFT      Move the video on the next tabulation point

CTRL-RIGHT     Move the video on previusly tabulation point.

F1             Display the available commands.

NOTE: tabulation is assumed of 8 characters.

The  << simbol  on the  left upper  corner of the viewer screen, if

present,  tell  you that the screen is shifted right, is not on the

first column of the line.

The viewer riquire about 9k bytes of free memory.

TSTHOST v1.42 (c) IK1GKJ                                     72

FILE AND MESSAGES EDITOR

------------------------

The  same editor  used to  edit a  file is  used to  edit  the  PMS

messages.  The editor  is structured  to edit  only TEXT  file, not

binary.

Editor available commands:

CURSOR UP/DWN       move to previusly/next line

PAGE UP-DOWN        move to previusly/next page

CTRL-HOME/END       move to top/bottom of editor

CTRL-PG UP/DWN      move to top/bottom of editor

CURSOR RIGHT/LEFT   move to next/previusly character

HOME/END            move to start/end of line

CTRL-RIGHT/LEFT     move to start of next/previusly word

F1   display a little help for the available commands

F2   save the contents of the editor and exit

F5   insert a file on current cursor position. Will be displayed

a list of files starting from the workdir directory: select

the desidered file with the enter key or abort the operation

with the escape key.

F8   Abort editing and exit. Before abort, will be asl to enter

Yes or No to confirm the abort.

BACKSPACE      Delete character left of the cursor

DELETE         Delete character under the cursor

CTRL-BS        Delete from cursor position to the start of the

left word.

CTRL-Y         Delete the current line

CTRL-W         Toggle on off word wrap mode

CTRL-J         Toggle on off justfication mode

INSERT         Toggle insert overwrite mode

RETURN         Go to end of line and eventually trunch the current

line if insert is active and the cursor is not on

the end of the line.

TAB            insert space to the next tabulation point if insert

is active, or move the cursor to te next tabulation

point if the overwrite mode is selected.

The status line diasplay this information:

I    Insert active, little cursor

W    Word wrap active

Fxx  Free memory left

Cxx Current cursor column

Lxx Current cursor line

TSTHOST v1.42 (c) IK1GKJ                                     73

WORD  WRAP: Word  wrap is  active on  column 78, adn is referred to

the  cursor postion, not the effective line lenght: in thi mode the

text  to right of the cursor is maintained to the original form and

you  do not  destroy ist formattation while correct the text to the

left.  For the  same reason,  word wrap  is active  ONLY in  insert

mode:  if you are in overwrite mode, the editor assume that you are

corretting a line, and do not touch its original format.

Again,  the word  warp is available only while insert a characters:

if  for example  you are  on a line that is more than 78 characters

size,  and you move the cursor without insert charaters, the editor

do  not split  the line,  so you  may examine a longh line with the

wrap active without destroy its format.

File  reading and  import do  not have word wrap: file while readed

will be maintained to its original formattation.

If  word wrap  is deactivated, you may edit line to unlimited size,

or  limited only  by the  free memory:  you must press enter key to

insert a new line.

In  overwrite mode,  all typed  will overwrite  the contents of the

previusly  data, with  an excpetion:  if you  are at the end of the

line,  and insert  new  data,  the  data  will  not  overwrite  the

contents  of the  next line  but will  be added  to the  end of the

current line.

Justification,  if active,  will be done by the wraparound routine,

so follow the same rules.

NOTE:  The editor  may handle  up to  24576 characters, and require

about 26K of memory to run.

TSTHOST v1.42 (c) IK1GKJ                                     74

MAINTUSR - program to manage user database

------------------------------------------

MAINTUSR  program is  supplied to  modify the  user  database  file

TSTHOST.USR:  any user  that connect  your pms  have a  record here

defined in this file.

Running  MAINTUSR you  will see  the list  of all  users that  have

connected  your pms.  In this  list  will  be  presented  the  main

information  about the  user: the name, the privated directory, the

date  of last  connection on  your system, the number of connection

made to your pms from the user.

Pressing  F1 key  the  programm  will  display  a  little  help  to

remember ypu the available options:

-  Using CURSOR KEY, or home, end, pg up, pg down, you may navigate

trought  the list.  The actual  selected record  is marked  by  the

highlighted line.

-  Pressing ENTER  you may  be able to modify the user name and his

private  directory. In the high part of the screen the program will

ask  to you  for enter  the user data: end the input with the ENTER

key  or abort  it with ESCAPE key. If you press enter without enter

any  data, the  field will  be empty,  this is  useful to  remove a

previusly  set private  directory. The max dimension for name field

is  12 characters,  while the  size  of  private  directory  is  30

characters,  that  will  be  reduce  to  29  because  the  programs

automatically add the '\' terminator.

-  ESCAPE permits to abort the database upadating. Maintusr program

will ask for a confirm: Y for yes or N for no.

-  With F10 key you will save the modified record and terminate the

program. Again, maintusr will aska confirm to you. (Y/N).

-  With ALT-D  combination key  you may  marked  as  "deleted"  the

current  record, or,  if the record is already deleted, restore it.

A  deleted record  will be  displayed with  a 'D' characters in the

first  column. Warning:  once saved  database, you  cannot restrore

deleted  record, because  thay will  be phisically removed from the

database.

-  With ALT-K  you may  automatically delete all the user that does

not  connect you pms from more than X days: the program will ask to

you  for the  specified number  of day;  pressing ENTER  to confirm

input data or ESCAPE to abort.

TSTHOST v1.42 (c) IK1GKJ                                     75

-  ALT-F is used to search a particoular callsign, or a part of it.

Maintuser  will ask  to input  the callsign to search for. Pressing

ENTER  will confirm  the input  data, while ESCAPE is used to abort

the  input callsign.  If found,  the  desidered  callsign  will  be

placed  under the  highlited bar.  The search  field will be always

start from the beginning of the file.

The  same command  ALT-F, if  any record  will be  found,  will  be

provided,  after operator  confirmation, to  the creation  of a new

record for the specified callsign.

-  ALT-N work similar to the ALT-F key, but is used to continue the

previusly search comparison from the currently selected record.

PROBLEMS

--------

Please   report  any   kind of   problem or  defect with  a message

to

IK1GKJ@IK1MSL, reporting:

- Defect type.

- Which operation you have done when you have noticed the defect.

- PC model and configuration

- Memory driver (EMM, QEMM etc.)

- PC Operating system version.

- (If possible) the TSR programs loaded in memory.

- TNC model and driver type

Thanks.

TSTHOST v1.42 (c) IK1GKJ                                     76

Summary of the arguments:

Features ........................................................1

Tsthost with TNC in KISS or BAYCOM modem  (/K) ..................3

Tsthost with the standard HOST mode  (/H) .......................4

Tsthost with DRSI driver  (/D) ..................................4

Tsthost with 2 or more tnc ......................................5

Memory usage ....................................................6

Expanded memory and /NOEMS command ...........................6

Extended memory and /NOXMS command ...........................6

Reduction of RX buffers, /Rsize command ......................7

Number of channels, /K parameters ............................7

Unproto list size, /U parameters .............................7

VGA mode ........................................................7

Extended interface for data request, switch /V ..................8

Short cut keys .................................................10

Path of the files ..............................................11

PG - Server directory .......................................11

MAIL - Messagges directory ..................................12

USERS - Users directory .....................................12

UPLOAD - Users upload directory .............................12

WORKDIR - Standard work directory ...........................12

TSTHOST.IRQ - Internal interrupt detection ..................12

TSTHOST.USR - Users database ................................13

TSTHOST.HBB - Automatic routing file ........................13

TSTHOST.PSW - Automatic password file .......................13

TSTHOST.SYS - Password file for PMS SYS command .............14

TSTHOST.CFG - Automatic configuration file ..................14

TSTHOST.SHD - Shutdown configuration file ...................14

???????.ULS - Unproto list file .............................15

TSTHOST.ENT - PMS connection text ...........................15

TSTHOST.HLP - PMS help file .................................15

TSTHOST.PGS - PMS PG server list ............................15

TSTHOST.LOG - Connections log ...............................15

SOURCE files ................................................15

TSTHOST.CRN - Command file executed every hours .............16

TSTHOST.BID - Bid file ......................................17

TSTHOST.IN - Mail import file ...............................18

TSTHOST.DMS - PMS messages database .........................19

TSTHOST.EXT - File extension for automatic execution ........20

TSTHOST.MAC AND THE MACRO - File definition for macro command21

What is a macro? ............................................21

Macro specified command .....................................22

TstHost.mac file format .....................................24

TSTHOST commands not active in the macro ....................25

TSTHOST commands that require an detailed arguments .........25

Macro example ...............................................26

AUTOMATIC PASSWORD .............................................28

IMPORTANT, PLEASE READ ......................................29

TSTHOST v1.42 (c) IK1GKJ                                     77

FORWARD,  UNPROTO  list,  CHANNEL  8  and automatic password ...30

???????.CON - HomeBbs connection file .......................30

FORWARD protocol ...............................................33

Problem to connect for UNPROTO and FORWARD .....................34

PMS INTERFACE ..................................................35

MAIL WORD FLAH IN SCREEN ....................................35

PRIVATE DIRECTORY FOR PMS USERS .............................36

THE SERVER ..................................................37

PG SERVER ...................................................37

CON_FILT.EXE, connection filter .............................39

MESSAGGE FLAGS ..............................................40

PMS REMOTE MESSAGE MANAGEMENT ...............................41

THE TZ ENVIROMENT VARIABLE ..................................41

PMS MESSAGE MANAGEMENT ......................................42

PMS COMMAND FROM KEYBOARD ...................................45

PMS commands for remote users ...............................50

UNPROTO LIST ...................................................53

IMPORTANT NOTE ..............................................54

UList Subcommands ...........................................55

TstHost commands description ...................................58

Send,Delete,Dir,Yput,View parameter window ..................58

Option that include a directory display .....................58

AX25 Subcommands ............................................65

THE STATUS LINE ................................................69

EXIT CODE ......................................................70

HOW PRINT A FILE ...............................................70

SMARTDRV, DISK CACHE ...........................................71

FILE AND MESSAGES VIEWER .......................................71

FILE AND MESSAGES EDITOR .......................................72

MAINTUSR - program to manage user database .....................74

PROBLEMS .......................................................75